|  | KONERU LAKSHMAIAH EDUCATION FOUNDATION (Deemed to be University estd, u/s, 3 of the UGC Act, 1956) (NAAC Accredited “A++” Grade University) Green Fields, Guntur District, A.P., India – 522502 Department of Electronics and Communication Engineering (DST - FIST Sponsored Department) |  |
| --- | --- | --- |

B.Tech.II ECE PROGRAM

A.Y.2023-24ODD, Semester-III

24EC22104 Analog Electronic Circuit Design

CO1

SESSION-1

Bipolar Junction Transistors (BJT)

1.Course Description (Description about the subject)

This course provides the student with the fundamental skills to apply the basics of semiconductor and devices like Bipolar junction transistor, FET, MOSFET and operational amplifier. It would build mathematical and numerical background for design of electronics circuit. Students equipped with the knowledge and skilling provided in the course would enable them to design, develop electronic circuits for real time applications.

2.Aim

The aim of this course is to make the students to apply the basics of semiconductor physics, outline the input and output characteristics of bipolar and unipolar transistors such as JFET and MOSFET, interpret its behavior under different biasing condition, discuss the characteristics of Op-amp, analyze its various linear, nonlinear applications, and characterize the design of an OP-amp filters, Oscillators, and its applications.

3. Instructional Objectives (Course Objectives)

- To impart knowledge on working of a bipolar transistor, its configuration, and its applications.

- To impart knowledge on the need of biasing, types of biasing and interpret its performance.

- To impart knowledge on working of a JFET and MOSFET and interpret its characteristics and parameters.

- To impart knowledge on operational amplifiers and analyse the characteristics of Op-Amp circuits to perform various linear and nonlinear applications.

- To impart knowledge on Operational amplifiers to perform as Active filters and Oscillators and analyse its applications.

- To familiarize with theoretical and mathematical aspects related to electronic circuits, such that student will be able to design electronic systems for real time applications.

4.Learning Outcomes (Course Outcome)

| CO.No. | Course Outcome (CO) | PO/PSO | BTL |
| --- | --- | --- | --- |
| CO 1 | Apply the knowledge of Semiconductor physics and discuss BJT configurations and its applications | PO1, PSO1 | 3 |
| CO 2 | Apply the limitations of BJT and discuss the characteristics and applications of Field Effect Transistors | PO1, PSO1 | 3 |
| CO 3 | Apply the linear and nonlinear circuits approaches and realise the characteristics of operational Amplifiers | PO1, PO2, PSO1 | 4 |
| CO 4 | Apply the concept of feedback system and realise the working principles of Oscillators and multivibrators | PO1, PO2, PSO1 | 4 |
| CO 5 | Design and analyse analog circuits for real time applications using Passive and Active Components. | PO2, PO3, PSO1 | 4 |
| CO 6 | Simulate and analyze Electronic Circuit using Multisim and myDAQ. | PO2, PO3, PSO1 | 4 |

5. Module Description (CO-1 Description)

This module provides the application and analysis the basic concept of BJT and biasing of BJT for amplification of signals

6. Session Outcomes

- Describe the working principle of BJT.

- Demonstrate the applications of BJT as amplifier.

7. Session Introduction

A bipolar junction transistor (BJT) is a three-terminal electronic device widely used for amplification and switching applications. It consists of two pn-junctions formed between three layers of semiconductor material: the emitter, base, and collector. The BJT operates by controlling the current flow between the collector and emitter terminals using a small current at the base terminal. It can function as either an NPN or PNP transistor. The BJT offers high current gain, making it suitable for amplifying weak signals. It is commonly employed in audio amplifiers, power supplies, digital logic circuits, and various other electronic systems that require signal amplification or switching functionality.

8. Session description

Bipolar junction transistor (BJT) is a bidirectional device that uses both electrons and holes as charge carriers. While Unipolar transistor i.e. field effect transistor uses only one type of charge carrier. BJT is a current controlled device. The current flows from emitter to collector or from collector to emitter depending on the type of connection. This main current is controlled by a very small current at the base terminal.

8.1 Construction

Bipolar junction transistor is formed by the combination of two back-to-back doped semiconductor materials. In other words, BJT is formed by the “sandwich” of back to back extrinsic semiconductor materials. These extrinsic semiconductors are PN junction diodes. Two PN junctions’ diodes are sandwiched together to form a three-terminal device knows as BJT transistor. BJT is a three-terminal device having two junctions.

After doping an intrinsic semiconductor with Trivalent or Pentavalent impurities a P-type semiconductor or N-type semiconductor respectively is made. If the number of electrons are greater than the number of holes (positive carriers) then that is known as N-type semiconductor material. While in P-type semiconductor, the number of holes is greater than the number of electrons. When P-type and N-type material are connected together then it becomes a PN-junction diode. BJT transistors are formed after connecting two PN junctions back to back. These transistors are known as PNP or NPN bipolar junction transistors depending on whether P or N-type is sandwiched.

Figure 1: PNP and NPN BJT

Basically, transistors have three portions and two junctions. These three portions are called Emitter, Collector, and Base. The emitter and collector sandwich the base in between them. The middle portion (base) forms two junctions with the emitter & collector. The junction of the base with emitter is known as the Emitter-Base junction while the junction of the base with the collector is known as the Collector-Base junction.

8.2 Terminals of BJT

There are three terminals of BJT. These terminals are knowns as collector, emitter and base. These terminals are briefly discussed here.

Emitter

The emitter is the portion on one side of the transistor which emits electrons or holes to the other two portions. The base is always reverse bias with respect to emitter so that it can emit a large number of majority carriers. It is the most heavily doped region of the BJT. The emitter-base junction should be always forward bias in both PNP and NPN transistors. Emitter supplies electrons to the emitter-base junction in NPN while it supplies holes into the same junction in PNP transistor.

Collector

The portion on the opposite side of the Emitter that collects the emitted charge carriers (i.e. electrons or holes) is known as collector. The collector is heavily doped but the doping level of the collector is in between the lightly doping level of base and heavily doped level of emitter. Collector-base junction should be always reversed biased in both PNP and NPN transistors. The reason for reverse biasing is to remove charge carriers (electrons or holes) from the collector-base junction. The collector of NPN transistor collects electrons emitted by emitter. While in PNP transistor, it collects holes emitted by emitter.

Base

The base is the middle portion between collector and emitter & it forms two PN junctions between them. The base is the most lightly doped portion of the BJT. Being the middle portion of the BJT allows it to control the flow of charge carriers between emitter and collector. The base-collector junction shows high resistance because this junction is reversed bias.

8.3 Type of BJT

This three-layer device formed by back-to-back connection has specific names. It can be weather PNP or NPN. Both connections are disused here briefly.

8.4 PNP Construction

Figure 2: PNP construction

In PNP bipolar transistor, the N-type semiconductor is sandwiched between two P-type semiconductors. PNP transistors can be formed by connecting cathodes of two diodes. The cathodes of the diodes are connected together at a common point known as base. While the anodes of the diodes that are on the opposite sides are known as the collector and the emitter.

The emitter-base junction is forward bias while collector-base junction is reverse bias. So, in PNP type current flows from emitter to collector. The emitter, in this case, is at high potential to both collector and base.

8.5 NPN Construction

NPN type is exactly opposite to PNP type. In NPN bipolar transistor, the P-type semiconductor is sandwiched between two N-type semiconductors. When the anodes of two diodes are connected together it forms an NPN transistor. The current will flow from the collector to emitter because the collector terminal is more positive than emitter in NPN connection.

Figure 3: NPN construction

9. Activities/ Case studies/ related to the session

“BJT Switching Circuit”

a) Build a BJT switching circuit using a BJT, resistors, and a power supply.

b) Connect an LED or a relay to the collector-emitter path of the BJT.

c) Apply digital logic signals (e.g., from a microcontroller or switches) to control the BJT's on/off state.

d) Observe the LED or relay behavior when the BJT switches between saturation and cutoff regions.

e) Measure and analyze the current and voltage characteristics during switching operations.

10. Examples & contemporary extracts of articles/ practices to convey the idea of the session

“Bipolar Transistors” by Michael Shur (source: https://www.sciencedirect.com)

Extract: A BIPOLAR junction transistor is a semiconductor device with three contacts to the three regions of the device of different conductivity types: emitter, base, and collector. The emitter and collector have one conductivity type (these regions are usually doped by donors; i.e., they are n type), and the base has the opposite conductivity type (the base is usually doped by acceptors; i.e., the base is p-type). The charge carriers are injected from the emitter into the

base. They diffuse across the base and reach the collector region. This injection, and hence the collector current, are strong functions of the base current, so that a small change in the base current causes a much larger change in the collector current. Therefore, the bipolar junction transistor has a large current gain, which is defined as the ratio of the collector current over the base current.

11. SAQ’s-Self Assessment Questions

1. In a BJT, the majority carriers in the base region are:

a) Electrons

b) Holes

c) Both electrons and holes

d) None of the above

2. The BJT terminal that controls the flow of current between the other two terminals is:

a) Base

b) Collector

c) Emitter

d) None of the above

3. The BJT current gain is also known as:

a) Transconductance

b) Voltage gain

c) Current amplification factor

d) Power gain

4. A BJT operates in the active region when:

a) The base-emitter junction is forward biased, and the base-collector junction is reverse biased

b) The base-emitter junction is reverse biased, and the base-collector junction is forward biased

c) Both the base-emitter and base-collector junctions are forward biased

d) Both the base-emitter and base-collector junctions are reverse biased

5. The base current in a BJT is typically __________ the collector current.

a) Greater than

b) Equal to

c) Less than

d) Unrelated to

6. The output characteristics of a BJT show the relationship between:

a) Collector current and base-emitter voltage

b) Collector current and collector-emitter voltage

c) Base current and base-emitter voltage

d) Base current and collector-emitter voltage

7. The BJT is classified into two main types: NPN and _______.

a) N-channel

b) P-channel

c) PNP

d) NMOS

8. The BJT is a _______ device.

a) Voltage-controlled

b) Current-controlled

c) Both voltage and current-controlled

d) None of the above

9. The BJT cutoff region corresponds to:

a) Zero base current and maximum collector current

b) Maximum base current and maximum collector current

c) Zero base current and zero collector current

d) Maximum base current and zero collector current

10. The BJT saturation region corresponds to:

a) Maximum base current and zero collector current

b) Zero base current and zero collector current

c) Zero base current and maximum collector current

d) Maximum base current and maximum collector current

11. Which type of BJT consists of two layers of p-type semiconductor sandwiching one layer of n-type semiconductor?

a) NPN

b) PNP

c) NMOS

d) PMOS

12. The nomenclature NPN or PNP refers to the:

a) Material used in the emitter region

b) Material used in the base region

c) Material used in the collector region

d) Material used in all three regions

13. In an NPN transistor, the arrow in the emitter symbol points:

a) Inward

b) Outward

c) Sideways

d) There is no arrow in an NPN transistor symbol

14. Which of the following is true about the construction of a BJT?

a) The base region is lightly doped compared to the emitter and collector regions.

b) The emitter region is lightly doped compared to the base and collector regions.

c) The collector region is lightly doped compared to the base and emitter regions.

d) The doping levels are the same in all three regions.

15. The thin layer of insulating material used to isolate the collector from the base in a BJT is called:

a) Base contact

b) Emitter contact

c) Collector contact

d) Oxide layer

16. The purpose of the base region in a BJT is to:

a) Provide majority carriers for conduction

b) Control the flow of majority carriers

c) Provide mechanical support to the device

d) Reduce the resistance of the collector region

17. The collector current in a BJT is primarily due to the flow of:

a) Electrons

b) Holes

c) Both electrons and holes

d) None of the above

18. Which of the following statements is true regarding the biasing of a BJT?

a) Forward bias is applied to the base-emitter junction, and reverse bias is applied to the base-collector junction.

b) Reverse bias is applied to the base-emitter junction, and forward bias is applied to the base-collector junction.

c) Both the base-emitter and base-collector junctions are forward biased.

d) Both the base-emitter and base-collector junctions are reverse biased.

19. The region of a BJT where the base-emitter junction is forward biased and the base-collector junction is reverse biased is called:

a) Active region

b) Cutoff region

c) Saturation region

d) Reverse-active region

20. Which type of BJT is commonly used in digital logic circuits?

a) NPN

b) PNP

c) Both NPN and PNP

d) Neither NPN nor PNP

12. Summary

Bipolar Junction Transistors (BJTs) are three-terminal electronic devices widely used for amplification and switching applications. They consist of three layers of semiconductor material, forming two pn-junctions: the emitter, base, and collector. BJTs can operate in two configurations: NPN (negative-positive-negative) and PNP (positive-negative-positive). By controlling a small current at the base terminal, the BJT allows a much larger current to flow between the collector and emitter terminals. This property makes it an effective amplifier. BJTs are commonly utilized in audio amplifiers, power supplies, digital logic circuits, and various electronic systems. They provide high current gain, making them suitable for applications requiring signal amplification or switching functionality.

13. Terminal Questions

- Explain the construction of BJT

- Draw the symbols of NPN and PNP transistor and explain the terminals

- What is a bipolar junction transistor (BJT) and how does it differ from other types of transistors?

- What are the three terminals of a BJT and how are they labeled?

- How does a BJT operate as an amplifier?

- What is the difference between an NPN and PNP BJT?

- How does the BJT provide current amplification?

- What is the role of the base current in a BJT?

- How is the BJT biased for proper operation?

- What are the different modes of operation for a BJT?

- What are the typical applications of BJTs in electronic circuits?

- What are the key parameters that characterize the performance of a BJT?

14. Case Studies (Co Wise)

Case Study: Design of a BJT Amplifier for Audio Applications

Overview: An audio equipment manufacturer is developing a new amplifier for high-quality audio reproduction. The design team decides to use a BJT amplifier configuration due to its low distortion, good linearity, and wide bandwidth characteristics.

15. Answer Key

1. b) Holes

2. a) Base

3. c) Current amplification factor

4. a) The base-emitter junction is forward biased, and the base-collector junction is reverse biased

5. c) Less than

6. b) Collector current and collector-emitter voltage

7. c) PNP

8. b) Current-controlled

9. c) Zero base current and zero collector current

10. a) Maximum base current and zero collector current

11. a) NPN

12. c) Material used in the collector region

13. b) Outward

14. a) The base region is lightly doped compared to the emitter and collector regions.

15. d) Oxide layer

16. b) Control the flow of majority carriers

17. b) Holes

18. a) Forward bias is applied to the base-emitter junction, and reverse bias is applied to the base-collector junction.

19. a) Active region

20. c) Both NPN and PNP

16. Glossary

Bipolar Junction Transistor: A three-terminal electronic device made of semiconductor material.

Emitter: The terminal through which majority charge carriers (electrons for NPN, holes for PNP) flow out.

Base: The terminal that controls the transistor's conductivity by varying the current or voltage.

Collector: The terminal that collects the majority charge carriers and allows current flow.

NPN: A BJT configuration where the majority charge carriers are electrons.

PNP: A BJT configuration where the majority charge carriers are holes.

17. References of books, sites, links

Text Books:

1. Robert L. Boylestad and Louis Nashelsky - “Electronic Devices and Circuit Theory”, 7th Edition, Prentice Hall Publication (1998).

2.Ramakanth A. Gaykwad – “Op-Amps and Linear IC Applications”, 4th Edition, Pearson Education (2015)

3. Thomas L. Floyd – “Electronic Devices – Electron Flow Version”, 4th Edition, Pearson Education (2001)

4. A.S. Sedra&K.C.Smith, Microelectronics Circuits, 7th Edition, Oxford University Press (2014)

Reference Books:

1. Jacob Millman - “Electronic Devices and Circuits”, 4th Edition, McGraw Hill Education (2015)

2. D Roy Choudhury and Shail B. Jain – “Linear Integrated Circuits”, 4th Edition, New Age International Pvt.Ltd. (2017)

3. David A. Bell - “Electronic Devices and Circuits”, 5th Edition, Oxford University Press (2008)

4. Behzad Razavi – “Design of Analog CMOS Integrated Circuit”,2nd Edition, McGraw Hill Education (2017)

5. M. S. Tyagi “Introduction to Semiconductor Materials and Devices”, 2nd Edition, John Wiley & Sons Inc (2008)

Weblinks/MOOCs:

1. https://nptel.ac.in/courses/108102095

2. https://www.coursera.org/learn/electronics?

18. Keywords Bipolar Junction Transistor

NPN transistor, PNP transistor, Emitter, Base, Collector, Current gain, Saturation, Cut-off

Biasing