|  | KONERU LAKSHMAIAH EDUCATION FOUNDATION (Deemed to be University estd, u/s, 3 of the UGC Act, 1956) (NAAC Accredited “A++” Grade University) Green Fields, Guntur District, A.P., India – 522502 Department of Electronics and Communication Engineering (DST - FIST Sponsored Department) |  |
| --- | --- | --- |

B.Tech.II ECE PROGRAM

A.Y.2023-24ODD, Semester-III

24EC22104 Analog Electronic Circuit Design

CO1

SESSION-10

Analysis of BJT amplifier

1.Course Description (Description about the subject)

This course provides the student with the fundamental skills to apply the basics of semiconductor and devices like Bipolar junction transistor, FET, MOSFET and operational amplifier. It would build mathematical and numerical background for design of electronics circuit. Students equipped with the knowledge and skilling provided in the course would enable them to design, develop electronic circuits for real time applications.

2.Aim

The aim of this course is to make the students to apply the basics of semiconductor physics, outline the input and output characteristics of bipolar and unipolar transistors such as JFET and MOSFET, interpret its behavior under different biasing condition, discuss the characteristics of Op-amp, analyze its various linear, nonlinear applications, and characterize the design of an OP-amp filters, Oscillators, and its applications.

3. Instructional Objectives (Course Objectives)

- To impart knowledge on working of a bipolar transistor, its configuration, and its applications.

- To impart knowledge on the need of biasing, types of biasing and interpret its performance.

- To impart knowledge on working of a JFET and MOSFET and interpret its characteristics and parameters.

- To impart knowledge on operational amplifiers and analyse the characteristics of Op-Amp circuits to perform various linear and nonlinear applications.

- To impart knowledge on Operational amplifiers to perform as Active filters and Oscillators and analyse its applications.

- To familiarize with theoretical and mathematical aspects related to electronic circuits, such that student will be able to design electronic systems for real time applications.

4.Learning Outcomes (Course Outcome)

| CO.No. | Course Outcome (CO) | PO/PSO | BTL |
| --- | --- | --- | --- |
| CO 1 | Apply the knowledge of Semiconductor physics and discuss BJT configurations and its applications | PO1, PSO1 | 3 |
| CO 2 | Apply the limitations of BJT and discuss the characteristics and applications of Field Effect Transistors | PO1, PSO1 | 3 |
| CO 3 | Apply the linear and nonlinear circuits approaches and realise the characteristics of operational Amplifiers | PO1, PO2, PSO1 | 4 |
| CO 4 | Apply the concept of feedback system and realise the working principles of Oscillators and multivibrators | PO1, PO2, PSO1 | 4 |
| CO 5 | Design and analyse analog circuits for real time applications using Passive and Active Components. | PO2, PO3, PSO1 | 4 |
| CO 6 | Simulate and analyse Electronic Circuit using Multisim and myDAQ. | PO2, PO3, PSO1 | 4 |

5. Module Description (CO-1 Description)

This module provides the application and analysis the basic concept of BJT and biasing of BJT for amplification of signals

6. Session Outcomes

- Describe the factors to analyse the operation of BJT amplifiers.

- Demonstrate the applications of BJT as amplifier.

7. Session Introduction

The analysis of BJT (Bipolar Junction Transistor) amplifiers involves examining and understanding the behavior and performance of the amplifier circuit. It includes determining the voltage gain, input and output impedances, and frequency response. The analysis typically starts with the small-signal model, where the BJT is represented by its hybrid-π model or h-parameters. The small-signal model enables the application of linear circuit analysis techniques to calculate the amplifier parameters. The analysis considers biasing conditions, load resistors, coupling and bypass capacitors, and the amplifier configuration. By analyzing the BJT amplifier, designers can gain insights into its functionality, performance limitations, and potential improvements, aiding in the design and optimization of amplifier circuits for various applications.

8. Session description

8.1 Amplifier Operation

AC Quantities

Lowercase italic subscripts are used to indicate ac quantities of rms, peak, and peak-to-peak currents and voltages: Ic , Ie , Ib , Vc and Vce (rms values are assumed unless otherwise stated).

Instantaneous quantities are represented by both lowercase letters and subscripts such as ic, ie, ib, and vce.

Figure 1: AC analysis

Resistances often have different values when a circuit is analyzed from an AC viewpoint as opposed to a DC viewpoint. Lowercase subscripts are used to identify ac resistance values. Resistance values internal to the transistor use a lowercase r' to show it is an AC resistance. An example is the internal ac emitter resistance, r'e

8.2 The Linear Amplifier

A linear amplifier provides amplification of a signal without any distortion so that the output signal is an exact amplified replica of the input signal. A transistor with a sinusoidal ac source capacitively coupled to the base through C1 and a load capacitively coupled to the collector through C2 is shown in Fig.

Figure 2: linear amplifier

The coupling capacitors block DC and thus prevent Rs and RL from changing the DC bias voltages at the base and collector. The capacitors ideally appear as shorts to the signal voltage. Vs causes Vb to vary sinusoidally above and below its DC bias level, VBQ. The resulting variation in Ib produces a larger variation in Ic because of the current gain of the transistor.

As Ic increases, the collector voltage decreases. Ic varies above and below its Q-point value, ICQ, in phase with Ib .

Vce varies above and below its Q-point value, VCEQ, out of phase with Vb . A transistor always produces a phase inversion between the base voltage and the collector voltage.

8.3 Transistor AC Models

r Parameter Transistor Model

Figure 3: r Parameter Transistor Model

The effect of the AC base resistance rb ' is usually small enough to neglect, so it can be replaced by a short. The AC collector resistance rc ' is usually several hundred kilohms and can be replaced by an open. The resulting simplified r-parameter equivalent circuit is shown in Fig.

Figure 4: r-parameter equivalent circuit

Transistor AC operation: A resistance appears between the emitter and base terminals. This is the resistance “seen” looking into the emitter for a forward-biased transistor. The collector acts as a dependent current source of αacIe  or, equivalently, βacIb (diamond-shaped symbol). These are shown with a transistor symbol in Fig.

The AC emitter resistance, re' is the most important of the r parameters for amplifier analysis.

8.4 Comparison of the AC Beta (βac) to the DC Beta (βDC)

Figure 5: Comparison of the AC Beta (βac) to the DC Beta (βDC)

9. Activities/ Case studies/related to the session

Activity: Designing a Common Emitter BJT Amplifier

Objective: Design a Common Emitter BJT amplifier with specific voltage gain and input/output impedance requirements.

Note: This activity provides a basic framework for designing a BJT amplifier. Additional considerations, such as thermal management, signal source impedance, and stability analysis, may be necessary depending on the specific design requirements.

10. Examples & contemporary extracts of articles/ practices to convey the idea of the session

Basic Electronics | BJT Amplifiers (Source: https://www.circuitbread.com)

Extract:

Designing a BJT (Bipolar Junction Transistor) amplifier involves determining the required specifications, selecting appropriate components, and configuring the amplifier circuit. Here are the key steps involved in the design process:

Determine the Amplifier Specifications, Choose the Amplifier Configuration, Common Emitter (CE), Common Base (CB), or Common Collector (CC) configuration, Select the configuration based on the desired voltage gain, input/output impedance, and application requirements, Biasing, Select Coupling and Decoupling Capacitors, Determine the Amplifier Capacitor Values, Choose the Load Resistor, Design the Biasing Network, Perform AC Analysis, Stability Analysis and Compensation, Verify Performance and Troubleshoot:

11. SAQ's-Self Assessment Questions

Certainly! Here are 20 multiple-choice questions (MCQs) on the design of BJT amplifiers:

1. The purpose of biasing in a BJT amplifier is to:

a) Increase the power output

b) Reduce distortion

c) Improve efficiency

d) All of the above

2. The most commonly used configuration for BJT amplifiers is:

a) Common Emitter (CE)

b) Common Base (CB)

c) Common Collector (CC)

d) Common Gate (CG)

3. The purpose of a coupling capacitor in a BJT amplifier is to:

a) Block DC voltage from the input signal

b) Improve the frequency response

c) Provide DC biasing for the transistor

d) Increase the voltage gain

4. The voltage gain of a BJT amplifier is primarily determined by:

a) The collector current (IC)

b) The base current (IB)

c) The load resistor

d) The transistor's beta (β) value

5. The purpose of an emitter resistor in a BJT amplifier is to:

a) Set the DC operating point

b) Increase the voltage gain

c) Provide negative feedback

d) Improve the stability

6. The purpose of bypass capacitors in a BJT amplifier is to:

a) Block DC voltage from the input signal

b) Improve the frequency response

c) Stabilize the DC operating point

d) Reduce the distortion

7. The load resistor in a BJT amplifier is connected to the:

a) Collector terminal

b) Base terminal

c) Emitter terminal

d) Ground terminal

8. The purpose of thermal compensation in a BJT amplifier is to:

a) Stabilize the DC operating point

b) Improve the voltage gain

c) Reduce power dissipation

d) Increase the bandwidth

9. The input impedance of a BJT amplifier is primarily determined by:

a) The base-emitter junction

b) The collector-emitter junction

c) The emitter resistor

d) The load resistor

10. The output impedance of a BJT amplifier is primarily determined by:

a) The base-emitter junction

b) The collector-emitter junction

c) The emitter resistor

d) The load resistor

11. The purpose of a feedback resistor in a BJT amplifier is to:

a) Provide DC biasing

b) Control the voltage gain

c) Block AC signals

d) Stabilize the DC operating point

12. The Miller effect in a BJT amplifier refers to:

a) The increase in effective input capacitance

b) The increase in effective output capacitance

c) The reduction in power dissipation

d) The reduction in distortion

13. The purpose of a bypass capacitor in the emitter resistor of a BJT amplifier is to:

a) Improve the frequency response

b) Stabilize the DC operating point

c) Increase the voltage gain

d) Reduce the distortion

14. The stability of a BJT amplifier can be improved by:

a) Using negative feedback

b) Using positive feedback

c) Increasing the power supply voltage

d) Decreasing the input impedance

15. The purpose of emitter degeneration in a BJT amplifier is to:

a) Increase the input impedance

b) Increase the output impedance

c) Improve the linearity

d) Reduce the power consumption

16. The input capacitance of a BJT amplifier is primarily determined by:

a) The base-emitter junction

b) The collector-emitter junction

c) The emitter resistor

d) The load resistor

17. The purpose of a collector resistor in a BJT amplifier is to:

a) Provide DC biasing

b) Control the voltage gain

c) Block AC signals

d) Stabilize the DC operating point

18. The purpose of a bypass capacitor in the collector resistor of a BJT amplifier is to:

a) Improve the frequency response

b) Stabilize the DC operating point

c) Increase the voltage gain

d) Reduce the distortion

19. The Miller capacitance in a BJT amplifier refers to:

a) The capacitance between the base and collector terminals

b) The capacitance between the base and emitter terminals

c) The capacitance between the collector and emitter terminals

d) The capacitance between the input and output terminals

20. The stability of a BJT amplifier can be affected by variations in:

a) Temperature

b) Power supply voltage

c) Transistor characteristics

d) All of the above

12. Summary

The analysis of a BJT (Bipolar Junction Transistor) involves examining and understanding its behavior in various amplifier circuits. It includes determining parameters such as voltage gain, input and output impedances, frequency response, and power dissipation. By using the small-signal model, which represents the BJT as a linear device, analysis techniques can be applied to calculate these parameters. The analysis takes into account biasing conditions, load resistors, coupling and bypass capacitors, and the amplifier configuration. It also considers factors such as temperature variations, noise, distortion, and stability. By analyzing the BJT, engineers can gain insights into its performance characteristics, limitations, and potential improvements, facilitating the design and optimization of amplifier circuits for specific applications.

13. Terminal Questions

- What is the purpose of BJT amplifier analysis?

- What are the key parameters analyzed in BJT amplifier analysis?

- What is the small-signal model used in BJT amplifier analysis?

- How is the voltage gain of a BJT amplifier calculated?

- How are the input and output impedances determined in BJT amplifier analysis?

- What is the significance of biasing conditions in BJT amplifier analysis?

- How do load resistors affect the performance of a BJT amplifier?

- What is the role of coupling and bypass capacitors in BJT amplifier analysis?

- How does the choice of amplifier configuration impact the analysis of a BJT amplifier?

- How is the frequency response of a BJT amplifier analyzed?

- What is the effect of temperature variations on the analysis of a BJT amplifier?

- How does negative feedback influence the analysis of a BJT amplifier?

- What are the limitations and trade-offs involved in BJT amplifier analysis?

- How are the emitter current, collector current, and base current analyzed in BJT amplifiers?

- How does the input capacitance affect the analysis of a BJT amplifier?

- What are the considerations for high-frequency analysis of BJT amplifiers?

- How is the power dissipation analyzed in a BJT amplifier circuit?

- What are the techniques for noise analysis in BJT amplifiers?

- How does distortion analysis play a role in BJT amplifier analysis?

- What are the methods for stability analysis in BJT amplifiers?

14. Case Studies (Co Wise)

Case Study: Design of a BJT Amplifier for Audio Applications

Overview: An audio equipment manufacturer is developing a new amplifier for high-quality audio reproduction. The design team decides to use a BJT amplifier configuration due to its low distortion, good linearity, and wide bandwidth characteristics.

15. Answer Key

1. d) All of the above

2. a) Common Emitter (CE)

3. a) Block DC voltage from the input signal

4. d) The transistor's beta (β) value

5. a) Set the DC operating point

6. b) Improve the frequency response

7. a) Collector terminal

8. a) Stabilize the DC operating point

9. a) The base-emitter junction

10. c) The emitter resistor

11. b) Control the voltage gain

12. a) The increase in effective input capacitance

13. a) Improve the frequency response

14. a) Using negative feedback

15. c) Improve the linearity

16. a) The base-emitter junction

17. d) Stabilize the DC operating point

18. a) Improve the frequency response

19. c) The capacitance between the collector and emitter terminals

20. d) All of the above

16. References of books, sites, links

Text Books:

1. Robert L. Boylestad and Louis Nashelsky - “Electronic Devices and Circuit Theory”, 7th Edition, Prentice Hall Publication (1998).

2.Ramakanth A. Gaykwad – “Op-Amps and Linear IC Applications”, 4th Edition, Pearson Education (2015)

3. Thomas L. Floyd – “Electronic Devices – Electron Flow Version”, 4th Edition, Pearson Education (2001)

4. A.S. Sedra&K.C.Smith, Microelectronics Circuits, 7th Edition, Oxford University Press (2014)

17. Reference Books:

1. Jacob Millman - “Electronic Devices and Circuits”, 4th Edition, McGraw Hill Education (2015)

2. D Roy Choudhury and Shail B. Jain – “Linear Integrated Circuits”, 4th Edition, New Age International Pvt.Ltd. (2017)

3. David A. Bell - “Electronic Devices and Circuits”, 5th Edition, Oxford University Press (2008)

4. Behzad Razavi – “Design of Analog CMOS Integrated Circuit”,2nd Edition, McGraw Hill Education (2017)

5. M. S. Tyagi “Introduction to Semiconductor Materials and Devices”, 2nd Edition, John Wiley & Sons Inc (2008)

Weblinks/MOOCs:

1. https://nptel.ac.in/courses/108102095

2. https://www.coursera.org/learn/electronics?

18. Keywords

BJT amplifier, analysis, voltage gain, input impedance, output impedance, frequency response, power dissipation, small-signal model, hybrid-π model, h-parameters, biasing conditions, load resistors, coupling capacitor, bypass capacitor, amplifier configuration, temperature variations, noise analysis, distortion analysis, stability analysis, emitter current, collector current, base current, input capacitance, high-frequency analysis, power dissipation, linear device.