|  | KONERU LAKSHMAIAH EDUCATION FOUNDATION (Deemed to be University estd, u/s, 3 of the UGC Act, 1956) (NAAC Accredited “A++” Grade University) Green Fields, Guntur District, A.P., India – 522502 Department of Electronics and Communication Engineering (DST - FIST Sponsored Department) |  |
| --- | --- | --- |

B.Tech.II ECE PROGRAM

A.Y.2023-24ODD, Semester-III

24EC22104 Analog Electronic Circuit Design

CO1

SESSION-2

Working of BJT

1.Course Description (Description about the subject)

This course provides the student with the fundamental skills to apply the basics of semiconductor and devices like Bipolar junction transistor, FET, MOSFET and operational amplifier. It would build mathematical and numerical background for design of electronics circuit. Students equipped with the knowledge and skilling provided in the course would enable them to design, develop electronic circuits for real time applications.

2.Aim

The aim of this course is to make the students to apply the basics of semiconductor physics, outline the input and output characteristics of bipolar and unipolar transistors such as JFET and MOSFET, interpret its behavior under different biasing condition, discuss the characteristics of Op-amp, analyze its various linear, nonlinear applications, and characterize the design of an OP-amp filters, Oscillators, and its applications.

3. Instructional Objectives (Course Objectives)

- To impart knowledge on working of a bipolar transistor, its configuration, and its applications.

- To impart knowledge on the need of biasing, types of biasing and interpret its performance.

- To impart knowledge on working of a JFET and MOSFET and interpret its characteristics and parameters.

- To impart knowledge on operational amplifiers and analyse the characteristics of Op-Amp circuits to perform various linear and nonlinear applications.

- To impart knowledge on Operational amplifiers to perform as Active filters and Oscillators and analyse its applications.

- To familiarize with theoretical and mathematical aspects related to electronic circuits, such that student will be able to design electronic systems for real time applications.

4.Learning Outcomes (Course Outcome)

| CO.No. | Course Outcome (CO) | PO/PSO | BTL |
| --- | --- | --- | --- |
| CO 1 | Apply the knowledge of Semiconductor physics and discuss BJT configurations and its applications | PO1, PSO1 | 3 |
| CO 2 | Apply the limitations of BJT and discuss the characteristics and applications of Field Effect Transistors | PO1, PSO1 | 3 |
| CO 3 | Apply the linear and nonlinear circuits approaches and realise the characteristics of operational Amplifiers | PO1, PO2, PSO1 | 4 |
| CO 4 | Apply the concept of feedback system and realise the working principles of Oscillators and multivibrators | PO1, PO2, PSO1 | 4 |
| CO 5 | Design and analyse analog circuits for real time applications using Passive and Active Components. | PO2, PO3, PSO1 | 4 |
| CO 6 | Simulate and analyse Electronic Circuit using Multisim and myDAQ. | PO2, PO3, PSO1 | 4 |

5. Module Description (CO-1 Description)

This module provides the application and analysis the basic concept of  BJT and biasing of BJT for amplification of signals

6. Session Outcomes

- Describe the working principle of BJT.

- Demonstrate the applications of BJT as amplifier.

7. Session Introduction

The Bipolar Junction Transistor (BJT) is a three-layer semiconductor device that operates based on the principles of both majority and minority charge carriers. It consists of two pn-junctions formed by three layers: the emitter, base, and collector. The working of a BJT involves the interaction of two types of charge carriers: electrons and holes. In an NPN transistor, the emitter is heavily doped with electrons (N-type), while the base is lightly doped with holes (P-type). The collector is also lightly doped with electrons (N-type). Conversely, in a PNP transistor, the doping polarities are reversed. When a voltage is applied between the emitter and base, the emitter injects electrons into the base region. The base region, being thin and lightly doped, allows only a small number of electrons to recombine with the available holes. The majority of electrons diffuse across the base and reach the collector, forming the collector current. By controlling the current or voltage at the base, the BJT's conductivity can be modulated. In an NPN transistor, a small base current allows a larger collector current to flow. This property enables the BJT to function as an amplifier. The BJT's operation relies on the physical phenomena of electron and hole diffusion, recombination, and the formation of depletion regions at the pn-junctions. These processes determine the transistor's behavior and characteristics, making it a fundamental building block in various electronic circuits.

8. Session description

8.1 BJT Characteristics and Types

Usually, BJTs will have three terminals and two p-n junctions. The three terminals are namely, base, emitter and collector. In BJT, only less current will flow between base and emitter terminals and a larger current will flow between collector and emitter terminals. Based on the doping BJTs are mainly classified into two types. They are PNP transistors and NPN transistors.

BJTs are classified into two types, PNP transistors and NPN transistors. Let us understand these types of transistors in detail.

8.2 NPN Transistor

In NPN transistors, one p-type semiconductor is placed between two n-type semiconductors and it forms the two p-n junctions. These NPN transistors are widely used in many electronic devices, mainly to amplify weak signals.  In the NPN transistors, the current flow will be usually from the emitter to the collector region.

8.3 PNP Transistor

In PNP transistors, one n-type semiconductor is sandwiched between the two p-type semiconductors and creates two p-n junctions. The PNP transistors are mainly used to control current flow through the circuit. Usually, the p-n junction is considered a diode. So, the transistors look like two crystal diodes connected in series. In the PNP transistor, the left side diode is known as the emitter-base diode. The right side diode is known as the collector-base diode.

8.4 Bipolar Junction Transistor Configurations

A BJT can be configured into three types, they are a common collector configuration, common base configuration and common emitter configuration. In the common base characteristics, the base of the transistor is grounded, then the emitter turns as input and the collector turns as output.

Input characteristics of CB ΔVCB  at constant:  Rin = ΔVBEΔ/IE

Output characteristics of CB  ΔIE  at constant:  Rout =ΔVCB/ΔIB

Current Transformer characteristics of CB ΔVCB  at constant: 𝞪 = ΔIC/ΔIB

In the common collector characteristics, the collector of the transistor is grounded, then the base turns as input and the emitter turns as output.

Input characteristics of CC ΔVCB  at constant:  Rin = ΔVCB/ΔIB

Output characteristics of CC ΔIB  at constant:  Rout = ΔVCE/ΔIB

Current Transformer characteristics of CC ΔVCE  at constant: 𝞪 = ΔIB/ΔIE

In the common-emitter characteristics,  the emitter of the transistor is grounded, then the base turns as input and the collector turns as output.

Input characteristics of CC  ΔVCE at constant:  Rin = ΔVBEΔ/IB

Output characteristics of CC  ΔIB  at constant:  Rout = ΔVCE /ΔIE

Current Transformer characteristics of CC ΔVCB  at constant: 𝞪 =ΔIC/ΔIB

8.5 BJT Working Principle

The NPN transistor is a biased active region. Here, the base-emitter junction is forward biased and the collector-base junction is reversed biased. So, the width of the depletion region of the base-emitter junction is small, while compared to the width of the collector-base junction.  The forward biased BE junction will reduce the barrier potential and help the current to flow from the emitter to the base.

Usually, the base of NPN transistors is thin and lightly doped, so it has fewer holes while compared with the electrons in the emitter.  The recombination of holes in the base with electrons in the emitter region will constitute the flow of the base current.  Usually, the direction of conventional current flow will remain opposed to the flow of electrons.

Then the remaining large number of electrons in the emitter will cross the reverse-biased collector junction in the form of collector current.

According to Kirchhoff's Current Law, the emitter current is equal to the sum of collector current and base current. Generally, the base current IB will remain small when compared to the emitter current IE and the collector current IC

IE = IC + IB

Figure 1: BJT working principle

The only major difference between the NPN and PNP transistors are their majority charge carriers. The majority charge carriers of NPN transistors are electrons and the majority charge carriers of PNP transistors are holes.  All other working principles and their doping ratio will remain the same for both NPN and PNP transistors.

In the transistor, if the collector current increases, then the collector junction temperature will increase. So, the resistance provided by the collector also gets reduced. As a result the collector current increases. This phenomenon is known as the thermal runway in BJT transistors.

9. Activities/ Case studies/related to the session

Use the common emitter configuration and draw the CB and CC configurations

10. Examples & contemporary extracts of articles/ practices to convey the idea of the session

“Bipolar Transistors” by Michael Shur (source: https://www.sciencedirect.com)

Extract: A BIPOLAR junction transistor is a semiconductor device with three contacts to the three regions of the device of different conductivity types: emitter, base, and collector. The emitter and collector have one conductivity type (these regions are usually doped by donors; i.e., they are n type), and the base has the opposite conductivity type (the base is usually doped by acceptors; i.e., the base is p-type). The charge carriers are injected from the emitter into the

base. They diffuse across the base and reach the collector region. This injection, and hence the collector current, are strong functions of the base current, so that a small change in the base current causes a much larger change in the collector current. Therefore, the bipolar junction transistor has a large current gain, which is defined as the ratio of the collector current over the base current.

11. SAQ's-Self Assessment Questions

Certainly! Here are 20 multiple-choice questions related to the working of a Bipolar Junction Transistor (BJT):

1. What does BJT stand for?

a) Bipolar Junction Transistor

b) Base Junction Transistor

c) Biased Junction Transistor

d) Buffer Junction Transistor

2. Which type of BJT has a P-type base and N-type emitter?

a) PNP

b) NPN

c) P-type

d) N-type

3. What is the current gain of a BJT known as?

a) Voltage gain

b) Power gain

c) Current transfer ratio

d) Transconductance

4. Which terminal of a BJT is responsible for controlling the current flow between the other two terminals?

a) Base

b) Emitter

c) Collector

d) Substrate

5. In an NPN transistor, which junction is forward biased?

a) Base-emitter junction

b) Base-collector junction

c) Emitter-collector junction

d) Base-emitter and base-collector junctions

6. What is the purpose of the base region in a BJT?

a) To provide majority charge carriers

b) To control the flow of current

c) To provide mechanical support

d) To reduce power dissipation

7. Which configuration provides the highest voltage gain in a BJT?

a) Common emitter

b) Common base

c) Common collector

d) Both common emitter and common collector have the same voltage gain

8. What happens when a small current flows into the base terminal of an NPN BJT?

a) The transistor goes into cutoff mode

b) The transistor goes into saturation mode

c) The transistor goes into active mode

d) The transistor becomes unstable

9. Which type of biasing is used to keep the base-emitter junction of a BJT forward biased?

a) Forward biasing

b) Reverse biasing

c) Self-biasing

d) Feedback biasing

10. What is the approximate range of base current in a BJT when it is in active mode?

a) Very high

b) Very low

c) Moderate

d) Equal to the collector current

11. Which of the following equations represents the current gain (beta) of a BJT?

a) Ic = beta * Ib

b) Ib = beta * Ic

c) Ie = beta * Ic

d) Ic = beta * Ie

12. What is the primary disadvantage of a common base configuration?

a) Low voltage gain

b) Low current gain

c) High input impedance

d) High output impedance

13. In a BJT, what is the purpose of the collector terminal?

a) To inject minority carriers into the base

b) To provide majority charge carriers

c) To amplify the input signal

d) To control the base current

14. Which region of a BJT has the thinnest depletion region?

a) Base

b) Collector

c) Emitter

d) None, all regions have equal depletion region thickness

15. What is the relationship between the collector current (Ic) and the base current (Ib) in an active mode BJT?

a) Ic = Ib

b) Ic ≈ Ib

c) Ic ≈ beta * Ib

d) Ic ≈ alpha * Ib

16. Which BJT configuration provides a phase reversal between the input and output signals?

a) Common emitter

b) Common base

c) Common collector

d) All configurations provide phase reversal

17. What is the primary cause of thermal runaway in a BJT?

a) Excessive base

current

b) Insufficient base current

c) High ambient temperature

d) Low ambient temperature

18. What is the purpose of the metal heat sink attached to a power BJT?

a) To provide electrical insulation

b) To enhance thermal conductivity

c) To reduce power dissipation

d) To protect the transistor from physical damage

19. What is the maximum voltage rating that should not be exceeded at the collector-emitter junction of a BJT?

a) Vbe

b) Vce(sat)

c) Vcb

d) Vceo

20. What is the primary advantage of a Darlington pair configuration?

a) High voltage gain

b) High current gain

c) Low power dissipation

d) Low input impedance

12. Summary

The Bipolar Junction Transistor (BJT) operates based on the interaction of majority and minority charge carriers in a three-layer semiconductor device. In an NPN transistor, the emitter is heavily doped with electrons, while the base is lightly doped with holes. The collector is lightly doped with electrons. When a voltage is applied between the emitter and base, electrons flow from the emitter to the base, forming the emitter current. The base current controls the conductivity of the transistor. The majority of electrons diffuses across the thin base region and reaches the collector, forming the collector current. By modulating the base current, the BJT can amplify small input signals. The working principle involves processes such as diffusion, recombination, and the formation of depletion regions at the pn-junctions. Biasing is used to establish the operating point of the transistor. Overall, the BJT's working principle enables it to serve as a fundamental component in various electronic applications.

13. Terminal Questions

- With neat sketch explain working of BJT

- Write short notes on BJT configurations

- What are the three layers of a BJT and how are they doped in an NPN transistor?

- How does the emitter current flow in a BJT and what is its significance?

- How does the base current control the transistor's conductivity?

- What happens when a voltage is applied between the collector and emitter terminals?

- What is the role of majority and minority charge carriers in the BJT's operation?

- How does the base region influence the flow of majority carriers?

- What is the significance of the depletion regions formed at the pn-junctions?

- How does the BJT exhibit amplification when used in an amplifier circuit?

- What factors affect the current gain and voltage gain of a BJT?

- How does biasing play a role in establishing the operating point of a BJT?

14. Case Studies (Co Wise)

Case Study: Design of a BJT Amplifier for Audio Applications

Overview: An audio equipment manufacturer is developing a new amplifier for high-quality audio reproduction. The design team decides to use a BJT amplifier configuration due to its low distortion, good linearity, and wide bandwidth characteristics.

15. Answer Key

1. a) Bipolar Junction Transistor

2. a) PNP

3. c) Current transfer ratio

4. a) Base

5. a) Base-emitter junction

6. b) To control the flow of current

7. a) Common emitter

8. c) The transistor goes into active mode

9. a) Forward biasing

10. c) Moderate

11. a) Ic = beta * Ib

12. b) Low current gain

13. b) To provide majority charge carriers

14. a) Base

15. c) Ic ≈ beta * Ib

16. a) Common emitter

17. a) Excessive base current

18. b) To enhance thermal conductivity

19. d) Vceo

20. b) High current gain

16 Glossary

Bipolar Junction Transistor (BJT): A three-layer semiconductor device that operates based on the interaction of majority and minority charge carriers.

NPN Transistor: A BJT configuration where the emitter is N-type, the base is P-type, and the collector is N-type.

PNP Transistor: A BJT configuration where the emitter is P-type, the base is N-type, and the collector is P-type.

Emitter: The region in a BJT where majority charge carriers are injected into the base.

Base: The region in a BJT that controls the transistor's conductivity by modulating the current or voltage.

Collector: The region in a BJT where majority charge carriers are collected and contribute to the collector current.

Majority Charge Carriers: The dominant type of charge carriers in a specific region of a BJT (electrons for N-type, holes for P-type).

Minority Charge Carriers: The less abundant type of charge carriers in a specific region of a BJT (holes for N-type, electrons for P-type).

Emitter Current: The current flowing from the emitter to the base in a BJT.

Base Current: The current flowing into the base terminal of a BJT, controlling its conductivity.

Collector Current: The current flowing from the collector to the emitter in a BJT.

17. References of books, sites, links

Text Books:

1. Robert L. Boylestad and Louis Nashelsky - “Electronic Devices and Circuit Theory”, 7th Edition, Prentice Hall Publication (1998).

2.Ramakanth A. Gaykwad – “Op-Amps and Linear IC Applications”, 4th Edition, Pearson Education (2015)

3. Thomas L. Floyd – “Electronic Devices – Electron Flow Version”, 4th Edition, Pearson Education (2001)

4. A.S. Sedra&K.C.Smith, Microelectronics Circuits, 7th Edition, Oxford University Press (2014)

Reference Books:

1. Jacob Millman - “Electronic Devices and Circuits”, 4th Edition, McGraw Hill Education (2015)

2. D Roy Choudhury and Shail B. Jain – “Linear Integrated Circuits”, 4th Edition, New Age International Pvt.Ltd. (2017)

3. David A. Bell - “Electronic Devices and Circuits”, 5th Edition, Oxford University Press (2008)

4. Behzad Razavi – “Design of Analog CMOS Integrated Circuit”,2nd Edition, McGraw Hill Education (2017)

5. M. S. Tyagi “Introduction to Semiconductor Materials and Devices”, 2nd Edition, John Wiley & Sons Inc (2008)

Weblinks/MOOCs:

1. https://nptel.ac.in/courses/108102095

2. https://www.coursera.org/learn/electronics?

18. Keywords

NPN transistor, PNP transistor, Common emitter configuration, Common base configuration

Common collector configuration, Input impedance, Output impedance, Voltage gain, Current gain.