|  | KONERU LAKSHMAIAH EDUCATION FOUNDATION (Deemed to be University estd, u/s, 3 of the UGC Act, 1956) (NAAC Accredited “A++” Grade University) Green Fields, Guntur District, A.P., India – 522502 Department of Electronics and Communication Engineering (DST - FIST Sponsored Department) |  |
| --- | --- | --- |

B.Tech.II ECE PROGRAM

A.Y.2023-24ODD, Semester-III

24EC22104 Analog Electronic Circuit Design

CO1

SESSION-3

BJT Biasing

1.Course Description (Description about the subject)

This course provides the student with the fundamental skills to apply the basics of semiconductor and devices like Bipolar junction transistor, FET, MOSFET and operational amplifier. It would build mathematical and numerical background for design of electronics circuit. Students equipped with the knowledge and skilling provided in the course would enable them to design, develop electronic circuits for real time applications.

2.Aim

The aim of this course is to make the students to apply the basics of semiconductor physics, outline the input and output characteristics of bipolar and unipolar transistors such as JFET and MOSFET, interpret its behavior under different biasing condition, discuss the characteristics of Op-amp, analyze its various linear, nonlinear applications, and characterize the design of an OP-amp filters, Oscillators, and its applications.

3. Instructional Objectives (Course Objectives)

- To impart knowledge on working of a bipolar transistor, its configuration, and its applications.

- To impart knowledge on the need of biasing, types of biasing and interpret its performance.

- To impart knowledge on working of a JFET and MOSFET and interpret its characteristics and parameters.

- To impart knowledge on operational amplifiers and analyse the characteristics of Op-Amp circuits to perform various linear and nonlinear applications.

- To impart knowledge on Operational amplifiers to perform as Active filters and Oscillators and analyse its applications.

- To familiarize with theoretical and mathematical aspects related to electronic circuits, such that student will be able to design electronic systems for real time applications.

4.Learning Outcomes (Course Outcome)

| CO.No. | Course Outcome (CO) | PO/PSO | BTL |
| --- | --- | --- | --- |
| CO 1 | Apply the knowledge of Semiconductor physics and discuss BJT configurations and its applications | PO1, PSO1 | 3 |
| CO 2 | Apply the limitations of BJT and discuss the characteristics and applications of Field Effect Transistors | PO1, PSO1 | 3 |
| CO 3 | Apply the linear and nonlinear circuits approaches and realise the characteristics of operational Amplifiers | PO1, PO2, PSO1 | 4 |
| CO 4 | Apply the concept of feedback system and realise the working principles of Oscillators and multivibrators | PO1, PO2, PSO1 | 4 |
| CO 5 | Design and analyse analog circuits for real time applications using Passive and Active Components. | PO2, PO3, PSO1 | 4 |
| CO 6 | Simulate and analyse Electronic Circuit using Multisim and myDAQ. | PO2, PO3, PSO1 | 4 |

5. Module Description (CO-1 Description)

This module provides the application and analysis the basic concept of BJT and biasing of BJT for amplification of signals

6. Session Outcomes

- Describe the various biasing techniques for BJT

- Demonstrate the BJT biasing

7. Session Introduction

Biasing of a Bipolar Junction Transistor (BJT) is the process of applying suitable voltages or currents to establish its operating point. It involves setting the base-emitter and base-collector junctions at desired biasing levels to ensure proper transistor behavior. Biasing is crucial to maintain the BJT in an active region where it can function as an amplifier or switch effectively. By selecting appropriate biasing voltages and resistors, the transistor's quiescent point can be established, ensuring stable and linear operation. Proper biasing prevents the BJT from operating in cutoff or saturation regions, optimizing its performance and allowing it to accurately amplify signals or perform switching functions.

8. Session description

Transistor Biasing is the process of setting a transistors DC operating voltage or current conditions to the correct level so that any AC input signal can be amplified correctly by the transistor

The steady state operation of a bipolar transistor depends a great deal on its base current, collector voltage, and collector current values. Therefore, if the transistor is to operate correctly as a linear amplifier, it must be properly biased around its operating point as improper transistor biasing will result in a distorted output.

Establishing the correct operating point requires the selection of bias resistors and load resistors to provide the appropriate input current and collector voltage conditions. The correct biasing point for a bipolar transistor, either NPN or PNP, generally lies somewhere between the two extremes of operation with respect to it being either “fully-ON” or “fully-OFF” along its DC load line. This central operating point is called the “Quiescent Operating Point”, or Q-point for short.

When a bipolar transistor is biased so that the Q-point is near the middle of its operating range, that is approximately halfway between cut-off and saturation, it is said to be operating as a Class-A amplifier. This mode of operation allows the output voltage to increase and decrease around the amplifiers Q-point without distortion as the input signal swings through one complete cycle. In other words, the output is available for the full 360o of the input cycle.

The correct biasing of the transistor is achieved using a process known commonly as Base Bias.

But before we start looking at the possible different transistor biasing arrangements, lets first remind ourselves of a basic single transistor circuit along with its voltages and currents as shown on the left.

The function of the “DC Bias level” is to correctly set the transistors Q-point by setting its Collector current ( IC ) to a constant and steady state value without any external input signal applied to the transistors Base. This steady-state or DC operating point is set by the values of the circuits DC supply voltage ( Vcc ) and the value of any biasing resistors connected the transistors Base terminal. Since the transistors Base bias currents are steady-state DC currents, the appropriate use of coupling and bypass capacitors will help block any biasing currents from other transistor stage affecting the bias conditions of the next. Base bias networks can be used for Common-base (CB), common-collector (CC) or common-emitter (CE) transistor configurations. In this simple transistor biasing tutorial we will look at the different biasing arrangements available for a Common Emitter Amplifier.

8.1 Common Emitter Transistor Biasing

One of the most frequently used biasing circuits for a transistor circuit is with the self-biasing of the emitter-bias circuit were one or more biasing resistors are used to set up the initial DC values for the three transistor currents, (IB), (IC) and (IE). The two most common forms of bipolar transistor biasing are: Beta Dependent and Beta Independent. Transistor bias voltages are largely dependent on transistor beta, ( β ) so the biasing set up for one transistor may not necessarily be the same for another transistor as their beta values may be different. Transistor biasing can be achieved either by using a single feed back resistor or by using a simple voltage divider network to provide the required biasing voltage.

### 8.2 Fixed Base Biasing a Transistor

Figure 1: Fixed Base Biasing a Transistor

The circuit shown is called as a “fixed base bias circuit”, because the transistors base current, IB remains constant for given values of Vcc, and therefore the transistors operating point must also remain fixed. This two resistor biasing network is used to establish the initial operating region of the transistor using a fixed current bias. This type of transistor biasing arrangement is also beta dependent biasing as the steady-state condition of operation is a function of the transistors beta β value, so the biasing point will vary over a wide range for transistors of the same type as the characteristics of the transistors will not be exactly the same. The emitter diode of the transistor is forward biased by applying the required positive base bias voltage via the current limiting resistor RB. Assuming a standard bipolar transistor, the forward base-emitter voltage drop would be 0.7V. Then the value of RB is simply: (VCC – VBE)/IB where IB is defined as IC/β. With this single resistor type of biasing arrangement the biasing voltages and currents do not remain stable during transistor operation and can vary enormously. Also the operating temperature of the transistor can adversely effect the operating point.

### 8.3 Collector Feedback Biasing

Figure 2: Collector Feedback Biasing

This self-biasing collector feedback configuration is another beta dependent biasing method which requires two resistors to provide the necessary DC bias for the transistor. The collector to base feedback configuration ensures that the transistor is always biased in the active region regardless of the value of Beta (β). The DC base bias voltage is derived from the collector voltage VC, thus providing good stability. In this circuit, the base bias resistor, RB is connected to the transistors collector C, instead of to the supply voltage rail, Vcc. Now if the collector current increases, the collector voltage drops, reducing the base drive and thereby automatically reducing the collector current to keep the transistors Q-point fixed. Therefore this method of collector feedback biasing produces negative feedback round the transistor as there is a direct feedback from the output terminal to the input terminal via resistor, RB. Since the biasing voltage is derived from the voltage drop across the load resistor, RL, if the load current increases there will be a larger voltage drop across RL, and a corresponding reduced collector voltage, VC. This effect will cause a corresponding drop in the base current, IB which in turn, brings IC back to normal. The opposite reaction will also occur when the transistors collector current reduces. Then this method of biasing is called self-biasing with the transistors stability using this type of feedback bias network being generally good for most amplifier designs.

8.4 Dual Feedback Transistor Biasing

Adding an additional resistor to the base bias network of the previous configuration improves stability even more with respect to variations in Beta, ( β ) by increasing the current flowing through the base biasing resistors. The current flowing through RB1 is generally set at a value equal to about 10% of collector current, IC. Obviously it must also be greater than the base current required for the minimum value of Beta, β.

One of the advantages of this type of self biasing configuration is that the two resistors provide both automatic biasing and Rƒ feedback at the same time.

Figure 3: Dual Feedback Transistor Biasing

### 8.5 Emitter Feedback Configuration

Figure 4: Emitter Feedback Configuration

This type of transistor biasing configuration, often called self-emitter biasing, uses both emitter and base-collector feedback to stabilize the collector current even further. This is because resistors RB1 and RE as well as the base-emitter junction of the transistor are all effectively connected in series with the supply voltage, VCC. The downside of this emitter feedback configuration is that it reduces the output gain due to the base resistor connection. The collector voltage determines the current flowing through the feedback resistor, RB1 producing what is called “degenerative feedback”. The current flowing from the emitter, IE (which is a combination of IC + IB) causes a voltage drop to appear across RE in such a direction, that it reverse biases the base-emitter junction.

So if the emitter current increases, due to an increase in collector current, voltage drop I*RE also increases. Since the polarity of this voltage reverse biases the base-emitter junction, IB automatically decrease. Therefore the emitter current increase less than it would have done had there been no self-biasing resistor. Generally, resistor values are set so that the voltage dropped across the emitter resistor RE is approximately 10% of VCC and the current flowing through resistor RB1 is 10% of the collector current IC. Thus this type of transistor biasing configuration works best at relatively low power supply voltages.

### 8.6 Voltage Divider Transistor Biasing

Figure 5: Voltage Divider Transistor Biasing

Here the common emitter transistor configuration is biased using a voltage divider network to increase stability. The name of this biasing configuration comes from the fact that the two resistors RB1 and RB2 form a voltage or potential divider network across the supply with their center point junction connected the transistors base terminal as shown. This voltage divider biasing configuration is the most widely used transistor biasing method. The emitter diode of the transistor is forward biased by the voltage value developed across resistor RB2. Also, voltage divider network biasing makes the transistor circuit independent of changes in beta as the biasing voltages set at the transistors base, emitter, and collector terminals are not dependent on external circuit values.

To calculate the voltage developed across resistor RB2 and therefore the voltage applied to the base terminal we simply use the voltage divider formula for resistors in series. Generally the voltage drop across resistor RB2 is much less than for resistor RB1. Clearly the transistors base voltage VB with respect to ground, will be equal to the voltage across RB2. The amount of biasing current flowing through resistor RB2 is generally set to 10 times the value of the required base current IB so that it is sufficiently high enough to have no effect on the voltage divider current or changes in Beta.

The goal of Transistor Biasing is to establish a known quiescent operating point, or Q-point for the bipolar transistor to work efficiently and produce an undistorted output signal. Correct DC biasing of the transistor also establishes its initial AC operating region with practical biasing circuits using either a two or four-resistor bias network.

In bipolar transistor circuits, the Q-point is represented by ( VCE, IC ) for the NPN transistors or ( VEC, IC ) for PNP transistors. The stability of the base bias network and therefore the Q-point is generally assessed by considering the collector current as a function of both Beta (β) and temperature.

9. Activities/ Case studies/related to the session

Design a BJT and determining the operating point.

The first step in BJT biasing is to determine the desired operating point or Q-point on the DC load line. This involves selecting the appropriate voltage and current levels for the transistor.

10. Examples & contemporary extracts of articles/ practices to convey the idea of the session

Transistor Biasing (source: www.electronics-tutorials.ws)

Extract: Transistor Biasing is the process of setting a transistors DC operating voltage or current conditions to the correct level so that any AC input signal can be amplified correctly by the transistor

Activities:

- An npn silicon transistor has VCC = 6 V and the collector load RC = 2.5 kΩ. Find: (i) The maximum collector current that can be allowed during the application of signal for faithful amplification. (ii) The minimum zero signal collector current required.

- A transistor employs a 4 kΩ load and VCC = 13V. What is the maximum input signal if β = 100 ? Given Vknee = 1V and a change of 1V in VBE causes a change of 5mA in collector current.

11. SAQ's-Self Assessment Questions

Certainly! Here are 20 multiple-choice questions (MCQs) on BJT biasing:

1. What is the purpose of biasing in a BJT circuit?

a) To provide stability to the operating point

b) To amplify the input signal

c) To protect the transistor from overheating

d) To provide mechanical support to the transistor

2. Which biasing method provides the highest stability for a BJT?

a) Fixed bias

b) Collector-to-base bias

c) Voltage divider bias

d) Emitter bias

3. What is the role of the biasing resistors in a BJT circuit?

a) To set the DC operating point

b) To amplify the signal

c) To provide negative feedback

d) To reduce power dissipation

4. Which biasing method requires two resistors and a voltage source?

a) Fixed bias

b) Collector-to-base bias

c) Voltage divider bias

d) Emitter bias

5. Which biasing method provides good stability and high input impedance?

a) Fixed bias

b) Collector-to-base bias

c) Voltage divider bias

d) Emitter bias

6. In a voltage divider bias configuration, which resistor sets the base bias voltage?

a) RB1

b) RB2

c) RE

d) RC

7. What happens to the operating point if the biasing resistors in a BJT circuit are increased?

a) The operating point moves towards saturation

b) The operating point moves towards cutoff

c) The operating point remains unchanged

d) The operating point becomes unstable

8. In a fixed bias configuration, where is the biasing voltage applied?

a) Base-emitter junction

b) Base-collector junction

c) Base terminal

d) Collector terminal

9. Which biasing method provides a relatively stable operating point but has a lower input impedance?

a) Fixed bias

b) Collector-to-base bias

c) Voltage divider bias

d) Emitter bias

10. Which biasing method is commonly used in integrated circuit (IC) designs?

a) Fixed bias

b) Collector-to-base bias

c) Voltage divider bias

d) Emitter bias

11. In an emitter bias configuration, what is the purpose of the bypass capacitor (CE)?

a) To stabilize the operating point

b) To provide negative feedback

c) To block DC and pass AC signals

d) To increase the gain of the circuit

12. Which biasing method is more immune to variations in transistor parameters and temperature?

a) Fixed bias

b) Collector-to-base bias

c) Voltage divider bias

d) Emitter bias

13. In a self-bias configuration, which component sets the biasing voltage?

a) Resistor

b) Capacitor

c) Diode

d) Transistor

14. Which biasing method provides the highest voltage gain in a BJT circuit?

a) Fixed bias

b) Collector-to-base bias

c) Voltage divider bias

d) Emitter bias

15. What happens to the operating point if the temperature increases in a BJT circuit?

a) The operating point moves towards saturation

b) The operating point moves towards cutoff

c) The operating point remains unchanged

d) The operating point becomes unstable

16. In a collector-to-base bias configuration, where is the biasing voltage applied?

a) Base-emitter junction

b) Base-collector junction

c) Base terminal

d) Collector terminal

17. What happens to the operating point if the biasing resistors in a BJT circuit are decreased?

a) The operating point moves towards saturation

b) The operating point moves towards cutoff

c) The operating point remains unchanged

d) The operating point becomes unstable

18. Which biasing method is most commonly used in amplifier circuits?

a) Fixed bias

b) Collector-to-base bias

c) Voltage divider bias

d) Emitter bias

19. In a BJT circuit, what is the primary purpose of the bypass capacitor in the emitter bias configuration?

a) To stabilize the operating point

b) To provide negative feedback

c) To block DC and pass AC signals

d) To increase the gain of the circuit

20. Which biasing method provides good stability, high input impedance, and relatively high voltage gain?

a) Fixed bias

b) Collector-to-base bias

c) Voltage divider bias

d) Emitter bias

12. Summary

Biasing of a Bipolar Junction Transistor (BJT) involves setting the appropriate voltages or currents to establish its operating point. The purpose of biasing is to ensure the BJT operates in its active region, where it can function as an amplifier or switch effectively. By selecting suitable biasing resistors and voltages, the quiescent point of the BJT is established, providing stable and linear operation. Proper biasing prevents the BJT from operating in cutoff or saturation regions, optimizing its performance. Biasing methods include fixed bias, emitter bias, and collector feedback bias, each with its advantages and considerations. Overall, proper biasing is essential for achieving reliable and desired BJT circuit behavior.

13. Terminal Questions

1. What is the purpose of biasing in a BJT circuit?

2. Explain the concept of quiescent point or operating point in BJT biasing.

3. What are the different biasing methods used for BJTs?

4. How does the choice of biasing method affect the stability and performance of a BJT circuit?

5. What is the significance of the DC load line in BJT biasing?

6. How can the biasing of a BJT be affected by temperature variations?

7. Discuss the advantages and disadvantages of using fixed biasing in BJT circuits.

8. What is the role of biasing resistors in a BJT circuit?

9. How does changing the biasing conditions affect the collector current and voltage of a BJT?

10. Explain the concept of bias stability and the factors that can impact it in BJT biasing.

14. Case Studies (Co Wise)

Case Study: Design of a BJT Amplifier for Audio Applications

Overview: An audio equipment manufacturer is developing a new amplifier for high-quality audio reproduction. The design team decides to use a BJT amplifier configuration due to its low distortion, good linearity, and wide bandwidth characteristics.

15. Answer Key

1. a) To provide stability to the operating point

2. c) Voltage divider bias

3. a) To set the DC operating point

4. c) Voltage divider bias

5. c) Voltage divider bias

6. b) RB2

7. b) The operating point moves towards cutoff

8. c) Base terminal

9. a) Fixed bias

10. c) Voltage divider bias

11. c) To block DC and pass AC signals

12. b) Collector-to-base bias

13. c) Diode

14. b) Collector-to-base bias

15. a) The operating point moves towards saturation

16. b) Base-collector junction

17. a) The operating point moves towards saturation

18. d) Emitter bias

19. c) To block DC and pass AC signals

20. d) Emitter bias

16. Glossary

Biasing: The process of setting the appropriate voltages or currents to establish the operating point of a BJT.

Quiescent Point: The operating point of a BJT where it is biased to operate in its desired region.

Active Region: The region of BJT operation where it functions as an amplifier or switch effectively.

Cutoff Region: The region of BJT operation where no collector current flows.

Saturation Region: The region of BJT operation where the collector current is at its maximum value.

Fixed Bias: A biasing method where a fixed voltage is applied to the base-emitter junction through a resistor.

Emitter Bias: A biasing method where the base-emitter junction is biased using a voltage divider network.

Collector Feedback Bias: A biasing method where the collector current is used to provide feedback for bias stabilization.

DC Load Line: A graphical representation of the possible operating points of a BJT on a current-voltage graph.

Bias Stability: The ability of the biasing circuit to maintain the desired operating point despite changes in temperature or transistor parameters.

Biasing Resistors: Resistors used in biasing circuits to control the voltage and current levels of the base-emitter and base-collector junctions.

Thermal Runaway: The phenomenon where an increase in temperature causes an increase in collector current, affecting bias stability.

Self-Biasing: A biasing technique where the biasing network is designed to be self-adjusting, compensating for changes in transistor parameters.

Bias Compensation: Techniques used to minimize the impact of temperature variations or transistor parameter changes on the biasing circuit.

Operating Point Stability: The ability of the biasing circuit to maintain a stable operating point despite variations in temperature, transistor gain, or power supply voltage.

17. References of books, sites, links

Text Books:

1. Robert L. Boylestad and Louis Nashelsky - “Electronic Devices and Circuit Theory”, 7th Edition, Prentice Hall Publication (1998).

2.Ramakanth A. Gaykwad – “Op-Amps and Linear IC Applications”, 4th Edition, Pearson Education (2015)

3. Thomas L. Floyd – “Electronic Devices – Electron Flow Version”, 4th Edition, Pearson Education (2001)

4. A.S. Sedra&K.C.Smith, Microelectronics Circuits, 7th Edition, Oxford University Press (2014)

Reference Books:

1. Jacob Millman - “Electronic Devices and Circuits”, 4th Edition, McGraw Hill Education (2015)

2. D Roy Choudhury and Shail B. Jain – “Linear Integrated Circuits”, 4th Edition, New Age International Pvt.Ltd. (2017)

3. David A. Bell - “Electronic Devices and Circuits”, 5th Edition, Oxford University Press (2008)

4. Behzad Razavi – “Design of Analog CMOS Integrated Circuit”,2nd Edition, McGraw Hill Education (2017)

5. M. S. Tyagi “Introduction to Semiconductor Materials and Devices”, 2nd Edition, John Wiley & Sons Inc (2008)

Weblinks/MOOCs:

1. https://nptel.ac.in/courses/108102095

2. https://www.coursera.org/learn/electronics?

18. Keywords

Biasing, Quiescent point, Active region, Cutoff region, Saturation region, Fixed bias, Emitter bias, Collector feedback bias, DC load line, Bias stability, Biasing resistors, Thermal runaway Self-biasing, Bias compensation, Operating point stability, Base-emitter junction, Base-collector junction, Voltage divider network, Transistor parameters, Temperature variations