|  | KONERU LAKSHMAIAH EDUCATION FOUNDATION (Deemed to be University estd, u/s, 3 of the UGC Act, 1956) (NAAC Accredited “A++” Grade University) Green Fields, Guntur District, A.P., India – 522502 Department of Electronics and Communication Engineering (DST - FIST Sponsored Department) |  |
| --- | --- | --- |

B.Tech.II ECE PROGRAM

A.Y.2023-24ODD, Semester-III

24EC22104 Analog Electronic Circuit Design

CO1

SESSION-5

Variation of operating point and its stability

1.Course Description (Description about the subject)

This course provides the student with the fundamental skills to apply the basics of semiconductor and devices like Bipolar junction transistor, FET, MOSFET and operational amplifier. It would build mathematical and numerical background for design of electronics circuit. Students equipped with the knowledge and skilling provided in the course would enable them to design, develop electronic circuits for real time applications.

2.Aim

The aim of this course is to make the students to apply the basics of semiconductor physics, outline the input and output characteristics of bipolar and unipolar transistors such as JFET and MOSFET, interpret its behavior under different biasing condition, discuss the characteristics of Op-amp, analyze its various linear, nonlinear applications, and characterize the design of an OP-amp filters, Oscillators, and its applications.

3. Instructional Objectives (Course Objectives)

- To impart knowledge on working of a bipolar transistor, its configuration, and its applications.

- To impart knowledge on the need of biasing, types of biasing and interpret its performance.

- To impart knowledge on working of a JFET and MOSFET and interpret its characteristics and parameters.

- To impart knowledge on operational amplifiers and analyse the characteristics of Op-Amp circuits to perform various linear and nonlinear applications.

- To impart knowledge on Operational amplifiers to perform as Active filters and Oscillators and analyse its applications.

- To familiarize with theoretical and mathematical aspects related to electronic circuits, such that student will be able to design electronic systems for real time applications.

4.Learning Outcomes (Course Outcome)

| CO.No. | Course Outcome (CO) | PO/PSO | BTL |
| --- | --- | --- | --- |
| CO 1 | Apply the knowledge of Semiconductor physics and discuss BJT configurations and its applications | PO1, PSO1 | 3 |
| CO 2 | Apply the limitations of BJT and discuss the characteristics and applications of Field Effect Transistors | PO1, PSO1 | 3 |
| CO 3 | Apply the linear and nonlinear circuits approaches and realise the characteristics of operational Amplifiers | PO1, PO2, PSO1 | 4 |
| CO 4 | Apply the concept of feedback system and realise the working principles of Oscillators and multivibrators | PO1, PO2, PSO1 | 4 |
| CO 5 | Design and analyse analog circuits for real time applications using Passive and Active Components. | PO2, PO3, PSO1 | 4 |
| CO 6 | Simulate and analyse Electronic Circuit using Multisim and myDAQ. | PO2, PO3, PSO1 | 4 |

5. Module Description (CO-1 Description)

This module provides the application and analysis the basic concept of  BJT and biasing of BJT for amplification of signals

6. Session Outcomes

- Describe the variation in operating point of BJT.

- Demonstrate the stability conditions in BJT.

7. Session Introduction

In BJT amplifiers, the operating point, also known as the quiescent point (Q-point), is a critical parameter that determines the transistor's biasing conditions. However, various factors can cause the operating point to deviate from its desired value, leading to variations and instability in amplifier performance. These factors include temperature variations, transistor parameter tolerances, and power supply fluctuations. When the operating point shifts, it can result in distorted output signals, reduced gain, and potential signal clipping. To ensure stability, proper biasing techniques such as voltage divider bias or emitter bias are employed, and techniques like thermal compensation and feedback are used to minimize the effects of variations and maintain a stable operating point.

8. Session description

8.1 Bias stability

Through proper biasing, a desired quiescent operating point of the transistor amplifier in the active region (linear region) of the characteristics is obtained. It is desired that once selected the operating point should remain stable. The maintenance of operating point stable is called Stabilization. The selection of a proper quiescent point generally depends on the following factors:

(a) The amplitude of the signal to be handled by the amplifier and distortion level in signal

(b) The load to which the amplifier is to work for a corresponding supply voltage

The operating point of a transistor amplifier shifts mainly with changes in temperature, since the transistor parameters — β, ICO and VBE (where the symbols carry their usual meaning)—are functions of temperature.

8.2 Requirements of biasing network

• Ensuring proper zero signal collector current.

• Ensuring VCE not falling below 0.5V for Ge transistor and 1V for Silicon transistor at any instant.

• Ensuring Stabilization of operating point. (zero signal IC and VCE)

8.3 The Thermal Stability of Operating Point (S)

Stability Factor S:- The stability factor S, as the change of collector current with respect to the reverse saturation current, keeping β and VBE constant. This can be written as:

The Thermal Stability Factor : SIco

SIco = ∂Ic/(∂Ico)

This equation signifies that Ic Changes SIco times as fast as Ico

Differentiating the equation of Collector Current IC = (1+β)Ico+ βIb & rearranging the terms we can write

SIco ═ (1+β)/ 1- β (∂Ib/∂IC)

It may be noted that Lower is the value of SIco better is the stability

8.4 Various Biasing Circuits

• Fixed Bias Circuit

• Fixed Bias with Emitter Resistor

• Collector to Base Bias Circuit

• Potential Divider Bias Circuit

8.5 The Fixed Bias Circuit

Figure 1: The Fixed Bias Circuit

The Thermal Stability Factor: SIco

SIco = (∂Ic/ ∂Ico) Vbe, β

General Equation of SIco Comes out to be

SIco ═ (1 + β)/ 1- β (∂Ib/∂IC)

Applying KVL through Base Circuit we can

write, Ib Rb+ Vbe= Vcc

Diff w. r. t. IC , we get (∂Ib/ ∂Ic) = 0

SIco= (1+β) is very large

Indicating high un-stability

Merits:

• It is simple to shift the operating point anywhere in the active region by merely changing the base resistor (RB).

• A very small number of components are required.

Demerits:

• The collector current does not remain constant with variation in temperature or power supply voltage. Therefore the operating point is unstable. • When the transistor is replaced with another one, considerable change in the value of β can be expected. Due to this change the operating point will shift.

• For small-signal transistors (e.g., not power transistors) with relatively high values of β (i.e., between 100 and 200), this configuration will be prone to thermal runaway. In particular, the stability factor, which is a measure of the change in collector current with changes in reverse saturation current, is approximately β+1. To ensure absolute stability of the amplifier, a stability factor of less than 25 is preferred, and so small-signal transistors have large stability factors.

9. Activities/ Case studies/related to the session

Activities:

- Fig. shows that a silicon transistor with β = 100 is biased by base resistor method. Draw the d.c. load line and determine the operating point. What is the stability factor ?

Figure Example 1

- It is desired to set the operating point at 2V, 1mA by biasing a silicon transistor with collector feedback resistor RB. If β = 100, find the value of RB.

Figure Example 2

10. Examples & contemporary extracts of articles/ practices to convey the idea of the session

DC Operating Point Study: A BJT Transistor (Source: resources.pcb.cadence.com)

Extract: The BJT transistor is perhaps one of the most interesting basic electronic components ever. Technically, it’s a combination of two PN junctions or diodes that are brought together in one package. A transistor functions as a current switch, where the presence of the base current will activate the collector-emitter (CE) junction for the collector current to pass through. There are a few interesting parameters when designing with a BJT transistor, one of them being the DC operating point. A DC operating point, also known as quiescent or Q point, refers to the state of the transistor when no input current is applied to the component.

11. SAQ's-Self Assessment Questions

Certainly! Here are 20 multiple-choice questions (MCQs) on the variation of the operating point and its stability in a Bipolar Junction Transistor (BJT) circuit:

1. The operating point of a BJT circuit refers to:

a) The point of maximum power dissipation

b) The point where the transistor is in saturation mode

c) The DC bias point of the transistor under normal operating conditions

d) The point where the transistor is in cutoff mode

2. The operating point of a BJT circuit is determined by:

a) The transistor's beta (β) value

b) The power supply voltage (VCC)

c) The collector-emitter voltage (VCE) and collector current (IC)

d) The base-emitter voltage (VBE)

3. What happens to the operating point if the base current (IB) is increased?

a) The operating point moves towards the saturation region

b) The operating point moves towards the cutoff region

c) The operating point remains unchanged

d) The operating point becomes unstable

4. The stability of the operating point in a BJT circuit is influenced by:

a) Temperature variations

b) Changes in transistor beta (β)

c) Variations in power supply voltage (VCC)

d) All of the above

5. What is the primary purpose of stabilizing the operating point in a BJT circuit?

a) To ensure maximum power dissipation

b) To achieve the highest possible gain

c) To prevent thermal runaway and ensure proper transistor operation

d) To shift the load line on the graph

6. The Q-point shift in a BJT circuit refers to:

a) Movement of the operating point along the load line

b) Changes in the transistor's beta (β) value

c) Variation in the power supply voltage (VCC)

d) Changes in the base-emitter voltage (VBE)

7. What happens if the operating point is located too close to the saturation region?

a) The transistor operates in cutoff mode

b) The transistor operates in active mode

c) The transistor operates in saturation mode

d) The Q-point becomes unstable

8. The stability factor (S) of a BJT circuit measures:

a) The variation in base-emitter voltage (VBE)

b) The variation in collector-emitter voltage (VCE)

c) The sensitivity of the Q-point to changes in transistor parameters

d) The gain of the BJT circuit

9. The load line in a BJT circuit represents the relationship between:

a) The base current (IB) and collector current (IC)

b) The collector current (IC) and collector-emitter voltage (VCE)

c) The base-emitter voltage (VBE) and base current (IB)

d) The collector-emitter voltage (VCE) and base-emitter voltage (VBE)

10. The stability of the operating point is improved by:

a) Increasing the power supply voltage (VCC)

b) Decreasing the collector-emitter voltage (VCE)

c) Adding bypass capacitors to the circuit

d) Increasing the base resistor (RB) value

11. The thermal stability of the operating point can be improved by:

a) Using a heat sink for the transistor

b) Decreasing the collector resistor (RC) value

c) Increasing the transistor's beta (β) value

d) Decreasing the power supply voltage (VCC)

12. Negative feedback in a BJT circuit helps to:

a) Stabilize the operating point

b) Increase the gain of the circuit

c) Decrease the power dissipation

d) Shift the load line on the graph

13. The stability factor (S) is calculated

using:

a) Transistor's beta (β) value

b) Transconductance (gm) of the transistor

c) Incremental collector current (ΔIC)

d) Incremental collector-emitter voltage (ΔVCE)

14. The thermal runaway phenomenon occurs when:

a) The operating point moves towards the cutoff region

b) The transistor's beta (β) decreases with temperature

c) The operating point moves towards the saturation region

d) The transistor's power dissipation exceeds the maximum rating

15. The stability of the operating point is crucial in amplifier circuits because:

a) It ensures maximum power dissipation

b) It determines the gain of the amplifier

c) It prevents distortion and instability in signal amplification

d) It affects the frequency response of the amplifier

16. The stability factor (S) of a BJT circuit should ideally be:

a) Zero

b) Less than 1

c) Greater than 1

d) Infinity

17. The operating point stability can be improved by:

a) Decreasing the base resistor (RB) value

b) Increasing the collector resistor (RC) value

c) Increasing the power supply voltage (VCC)

d) Reducing the transistor's beta (β) value

18. The stability factor (S) is a measure of:

a) The Q-point's position on the load line

b) The Q-point's sensitivity to variations in transistor parameters

c) The Q-point's proximity to the saturation region

d) The Q-point's distance from the cutoff region

19. The stability of the operating point is more critical in:

a) Low-frequency circuits

b) High-frequency circuits

c) Linear amplifiers

d) Digital logic circuits

20. The thermal stability of the operating point can be assessed by:

a) Performing temperature rise tests on the transistor

b) Calculating the thermal resistance of the transistor package

c) Analyzing the variation in the transistor's beta (β) value with temperature

d) Observing the variation in the collector-emitter voltage (VCE) with temperature

12. Terminal Questions

- What is the operating point of a BJT amplifier, and why is it important?

- What factors can cause variations in the operating point of a BJT amplifier?

- How do temperature variations affect the stability of the operating point?

- What are the consequences of an unstable operating point in a BJT amplifier?

- What is meant by transistor parameter tolerances, and how do they impact the operating point stability?

- How can power supply fluctuations influence the operating point stability?

- What biasing techniques are commonly used to establish a stable operating point?

- What is voltage divider bias, and how does it help maintain a stable operating point?

- Explain the concept of thermal compensation and its role in stabilizing the operating  point.

- How does feedback contribute to operating point stability in a BJT amplifier?

- What are the advantages and disadvantages of different biasing techniques concerning operating point stability?

- Can variations in the operating point affect the gain of a BJT amplifier? If so, how?

- How can the operating point stability be tested and measured in a BJT amplifier circuit?

- Are there any specific applications or scenarios where maintaining a stable operating point is particularly crucial?

- How does the selection of biasing resistors impact the stability of the operating point?

- What are the potential consequences of an over-biased or under-biased operating point?

- How can negative feedback be utilized to enhance the stability of the operating point?

- Is it possible to compensate for transistor parameter variations to maintain a stable operating point? If so, how?

- What techniques can be employed to mitigate the effects of power supply ripple on the operating point stability?

- How does the use of active components, such as temperature-compensated biasing circuits, contribute to operating point stability in BJT amplifiers?

13. Summary

The variation of the operating point and its stability in BJT amplifiers is a crucial aspect of circuit design. The operating point, or Q-point, determines the biasing conditions of the transistor and influences the amplifier's performance. However, factors like temperature changes, transistor parameter tolerances, and power supply fluctuations can cause the operating point to deviate, leading to distortion, reduced gain, and other issues. To ensure stability, various techniques are employed, including proper biasing methods like voltage divider bias and thermal compensation. Feedback mechanisms and careful component selection also play a role in maintaining a stable operating point. By addressing variations and maintaining stability, the amplifier can deliver reliable and consistent performance.

14. Case Studies (Co Wise)

Case Study: Design of a BJT Amplifier for Audio Applications

Overview: An audio equipment manufacturer is developing a new amplifier for high-quality audio reproduction. The design team decides to use a BJT amplifier configuration due to its low distortion, good linearity, and wide bandwidth characteristics.

15. Answer Key

1. c) The DC bias point of the transistor under normal operating conditions

2. c) The collector-emitter voltage (VCE) and collector current (IC)

3. c) The operating point remains unchanged

4. d) All of the above

5. c) To prevent thermal runaway and ensure proper transistor operation

6. a) Movement of the operating point along the load line

7. c) The transistor operates in saturation mode

8. c) The sensitivity of the Q-point to changes in transistor parameters

9. b) The collector current (IC) and collector-emitter voltage (VCE)

10. c) Adding bypass capacitors to the circuit

11. a) Using a heat sink for the transistor

12. a) Stabilize the operating point

13. d) Incremental collector-emitter voltage (ΔVCE)

14. d) The transistor's power dissipation exceeds the maximum rating

15. c) It prevents distortion and instability in signal amplification

16. b) Less than 1

17. b) Increasing the collector resistor (RC) value

18. b) The Q-point's sensitivity to variations in transistor parameters

19. a) Low-frequency circuits

20. c) Analyzing the variation in the transistor's beta (β) value with temperature

16. Glossary

Operating point: Also known as the Q-point, it represents the biasing conditions of a transistor in an amplifier circuit. It determines the voltage and current levels at which the transistor operates.

Stability: Refers to the ability of the amplifier to maintain a consistent operating point despite external influences. A stable operating point ensures predictable and reliable performance.

Variation: Refers to changes or deviations in the operating point caused by factors such as temperature fluctuations, transistor parameter tolerances, and power supply variations.

Temperature compensation: Techniques used to minimize the impact of temperature changes on the operating point. This helps maintain stability in the presence of temperature variations.

Transistor parameter tolerances: The natural variations in transistor characteristics that occur during manufacturing. These tolerances can affect the operating point and must be considered for stability.

Power supply fluctuations: Changes in the voltage supplied to the amplifier circuit that can cause the operating point to shift. Stable power supply regulation is important to minimize these fluctuations.

Biasing techniques: Methods used to establish a stable operating point, such as voltage divider bias, emitter bias, or feedback bias. These techniques provide the necessary biasing conditions for stable operation.

17. References of books, sites, links

Text Books:

1. Robert L. Boylestad and Louis Nashelsky - “Electronic Devices and Circuit Theory”, 7th Edition, Prentice Hall Publication (1998).

2.Ramakanth A. Gaykwad – “Op-Amps and Linear IC Applications”, 4th Edition, Pearson Education (2015)

3. Thomas L. Floyd – “Electronic Devices – Electron Flow Version”, 4th Edition, Pearson Education (2001)

4. A.S. Sedra&K.C.Smith, Microelectronics Circuits, 7th Edition, Oxford University Press (2014)

Reference Books:

1. Jacob Millman - “Electronic Devices and Circuits”, 4th Edition, McGraw Hill Education (2015)

2. D Roy Choudhury and Shail B. Jain – “Linear Integrated Circuits”, 4th Edition, New Age International Pvt.Ltd. (2017)

3. David A. Bell - “Electronic Devices and Circuits”, 5th Edition, Oxford University Press (2008)

4. Behzad Razavi – “Design of Analog CMOS Integrated Circuit”,2nd Edition, McGraw Hill Education (2017)

5. M. S. Tyagi “Introduction to Semiconductor Materials and Devices”, 2nd Edition, John Wiley & Sons Inc (2008)

Weblinks/MOOCs:

1. https://nptel.ac.in/courses/108102095

2. https://www.coursera.org/learn/electronics?

18. Keywords

Operating point, stability, BJT amplifiers, biasing conditions, Q-point, transistor parameters, temperature variations, power supply fluctuations, feedback, distortion, gain, linearity, compensation, linearity, voltage, current, feedback compensation.