|  | KONERU LAKSHMAIAH EDUCATION FOUNDATION (Deemed to be University estd, u/s, 3 of the UGC Act, 1956) (NAAC Accredited “A++” Grade University) Green Fields, Guntur District, A.P., India – 522502 Department of Electronics and Communication Engineering (DST - FIST Sponsored Department) |  |
| --- | --- | --- |

B.Tech.II ECE PROGRAM

A.Y.2023-24ODD, Semester-III

24EC22104 Analog Electronic Circuit Design

CO1

SESSION-6

Small signal models

1.Course Description (Description about the subject)

This course provides the student with the fundamental skills to apply the basics of semiconductor and devices like Bipolar junction transistor, FET, MOSFET and operational amplifier. It would build mathematical and numerical background for design of electronics circuit. Students equipped with the knowledge and skilling provided in the course would enable them to design, develop electronic circuits for real time applications.

2.Aim

The aim of this course is to make the students to apply the basics of semiconductor physics, outline the input and output characteristics of bipolar and unipolar transistors such as JFET and MOSFET, interpret its behavior under different biasing condition, discuss the characteristics of Op-amp, analyze its various linear, nonlinear applications, and characterize the design of an OP-amp filters, Oscillators, and its applications.

3. Instructional Objectives (Course Objectives)

- To impart knowledge on working of a bipolar transistor, its configuration, and its applications.

- To impart knowledge on the need of biasing, types of biasing and interpret its performance.

- To impart knowledge on working of a JFET and MOSFET and interpret its characteristics and parameters.

- To impart knowledge on operational amplifiers and analyse the characteristics of Op-Amp circuits to perform various linear and nonlinear applications.

- To impart knowledge on Operational amplifiers to perform as Active filters and Oscillators and analyse its applications.

- To familiarize with theoretical and mathematical aspects related to electronic circuits, such that student will be able to design electronic systems for real time applications.

4.Learning Outcomes (Course Outcome)

| CO.No. | Course Outcome (CO) | PO/PSO | BTL |
| --- | --- | --- | --- |
| CO 1 | Apply the knowledge of Semiconductor physics and discuss BJT configurations and its applications | PO1, PSO1 | 3 |
| CO 2 | Apply the limitations of BJT and discuss the characteristics and applications of Field Effect Transistors | PO1, PSO1 | 3 |
| CO 3 | Apply the linear and nonlinear circuits approaches and realise the characteristics of operational Amplifiers | PO1, PO2, PSO1 | 4 |
| CO 4 | Apply the concept of feedback system and realise the working principles of Oscillators and multivibrators | PO1, PO2, PSO1 | 4 |
| CO 5 | Design and analyse analog circuits for real time applications using Passive and Active Components. | PO2, PO3, PSO1 | 4 |
| CO 6 | Simulate and analyse Electronic Circuit using Multisim and myDAQ. | PO2, PO3, PSO1 | 4 |

5. Module Description (CO-1 Description)

This module provides the application and analysis the basic concept of BJT and biasing of BJT for amplification of signals

6. Session Outcomes

- Describe the types of small signal models of BJT.

- Designing amplifiers using small signal models.

7. Session Introduction

The small-signal models of a bipolar junction transistor (BJT) are simplified representations used to analyze the behavior of the transistor under small variations or signals around its operating point. These models provide a linearized view of the BJT, making it easier to analyze and design amplifier circuits. The small-signal models consider the BJT as a linear device and neglect non-linear effects. Two commonly used small-signal models for BJTs are the hybrid-π (pi) model and the h-parameter model. The hybrid-π model represents the BJT as a combination of resistors and controlled current and voltage sources. It is useful for analyzing voltage-driven amplifier configurations. The h-parameter model represents the BJT as a combination of input and output resistances and current gains. It is often used in current-driven amplifier configurations.

8. Session description

Analog circuits often operate with signal levels that are small compared to the bias currents and voltages in the circuit. Under this condition, incremental or small-signal models can be derived that allow calculation of circuit gain and terminal impedances without the necessity of including the bias quantities.

There are two popular techniques used to analyze BJT amplifier circuits. One is through the use of hybrid parameters. There are four different hybrid parameters. We have already seen one of them, the forward current gain,  hfe. We simply call it  β . The other three are  hie  , the input impedance;  hoe  , the output admittance; and  hre  , the reverse voltage gain. The second letter of the subscript (the “ e  ” in  hfe) indicates it is for the common emitter configuration (that is, input applied to the base, output taken at the collector and the emitter at the common ground).

The second approach uses r' parameters (pronounced “r prime”). The  r′   approach is sufficient for all of our analyses and, given an understanding of Ohm's law, KVL and KCL, produces straightforward equations for circuit gain, input impedance and the like. As a consequence, we shall focus on the  r′  system.

AC BJT model for our AC analyses. In fact, our AC model is based on the DC model. The collector-base region is still represented with a current-controlled current source although it's AC instead of DC:  iC=βiB

The base-emitter junction is a bit trickier. Although a simple 0.7 volt junction worked fine for DC, we now have to consider the AC resistance of the diode.

To find the dynamic resistance of the junction, first recall that the AC signal is riding on the DC bias current, as plotted in Figure. We can imagine that the AC signal is causing this point to trace back and forth along the curve. Of course, as this is a small signal analysis, this sweep will be very small, perhaps only a few percent of the quiescent current and can be approximated as a straight line segment. The slope of this line segment represents its conductance. The reciprocal of conductance is resistance; therefore, the reciprocal of the slope represents the resistance of the device. Consequently, we can approximate the dynamic resistance of the device as the reciprocal of the slope of the line tangent to the operating point (that is, the reciprocal of the slope of the line tangent to the quiescent bias current IC). In reality, this slope is changing slightly as the signal swings back and forth along the base-emitter I-V curve. As the signal swings positive and goes above the quiescent point the slope is a little steeper producing a slight reduction in dynamic resistance. In contrast, as it swings negative, going below the quiescent point, the slope becomes a bit more shallow and produces a slightly higher resistance. As a result, we are effectively computing an average value for the dynamic resistance by assuming this is a straight line segment.

Figure 1: V-I characteristics

In order to derive an equation for the dynamic resistance, we begin with the Shockley equation, slightly modified to reflect the terminal names of a BJT.

IC=IS(eVBEqnkT−1)

Where

IC   is the junction (collector) current,

IS  is the reverse saturation current,

VBE  is the voltage across the junction (base-emitter),

q  is the charge on an electron, 1.6E−19 coulombs,

n  is the quality factor (typically between 1 and 2),

k  is the Boltzmann constant, 1.38E−23 joules/kelvin,

T  is the temperature in kelvin.

At 300 kelvin (about 80 ∘  F),  q/kT   is approximately 38.6, thus for any reasonable value of  VBE   the “−1” term is small enough to ignore. Also, we shall taken  as 1.

The equation then reduces to IC=ISe38.6VBE(7.2.1)

To find the slope we take the first derivative of above Equation with respect to  VBE

.

dIC/dVBE=38.6ISe38.6VBE  yields

dIC dVBE=38.6IC

By definition, the dynamic junction resistance is the reciprocal of the slope.

dVBEdIC=25.9mVIC

We call this  r′e . This value is slightly low as it doesn't include bulk resistance so a good approximation is r′e=26mVIC. It is important to note that  IS  varies with temperature. Therefore  r′e varies with temperature as well, decreasing with increasing temperature. This carries important ramifications with the thermal stability of higher power amplifiers as we shall see in subsequent work. One of the most important things to remember here is that the DC collector current sets up the resistance of the AC model. In other words, the stability of the AC circuit will depend in part on the stability of the DC bias.

Figure 2: Simplified model

This is a simplified model in that it does not include junction capacitance effects, lead inductance and the like. It is appropriate, therefore, as a low to mid-band frequency model. To summarize, the AC collector current,  iC  , is determined by the AC input current,  iB  ; which is in turn a function of the the size of the applied input signal. In contrast,  r′e   is set by the DC bias current,  IC  . The AC input can produce small variations in  r′e  which are manifested as waveform distortion.

Given the model, there are three ways to configure the transistor as an amplifier:

Common Emitter. The input is applied to the base and the output is taken at the collector. The emitter terminal is at the common or ground point. This configuration exhibits both voltage gain and current gain. It also inverts the phase of the signal.

Common Collector. The input is applied to the base and the output is taken at the emitter. The collector terminal is at the common or ground point. This configuration offers a voltage gain of about unity but does exhibit current gain. It maintains the phase of the input signal. It is also referred to as an emitter follower or voltage follower.

Common Base. The input is applied to the emitter and the output is taken at the collector. The base terminal is at the common or ground point. This configuration exhibits voltage gain but the current gain is unity at best. It also maintains the phase of the input signal.

9. Activities/ Case studies/related to the session

- Draw the equivalent DC and small signal model of BJT in

10. Examples & contemporary extracts of articles/ practices to convey the idea of the session

BJT Small Signal Analysis (Source: https://www.academia.edu)

Extract: BJT Small Signal Analysis In the last classes we have seen how biasing of the transistor is done and we were considering different biasing schemes. All these biasing schemes are basically for using the transistor as an amplifier for faithful amplification of an input signal. Today we will discuss about the transistor being used as an amplifier and to understand the operation of the transistor as an amplifier we have to first understand the analysis of the BJT when a small signal is applied. So small signal analysis of a BJT will be discussed today. The small signal means the magnitude of the AC signal which is applied should be small enough to keep the transistor still in the active region of operation because we want the transistor to operate in the active region. Only then the transistor will be able to faithfully amplify an AC input signal. If that signal amplitude is very high then it is seen that the transistor will be driven into either saturation or cut off region because when you have very large signal at the input then the base current signal will be having a high magnitude. Peak to peak value of the base current will be high.

11. SAQ's-Self Assessment Questions

Certainly! Here are 20 multiple-choice questions (MCQs) on BJT small signal models:

1. Small signal models are used to analyze the behavior of a BJT circuit under:

a) DC bias conditions

b) AC signal conditions

c) Both DC and AC conditions

d) Transient conditions

2. The small signal model of a BJT is based on which linearization technique?

a) Small signal approximation

b) Large signal approximation

c) Piecewise linear approximation

d) Time-domain approximation

3. The small signal current gain of a BJT is denoted by:

a) α

b) β

c) γ

d) δ

4. In the small signal model, the input impedance is represented by:

a) rπ

b) gm

c) ro

d) Rc

5. The small signal voltage gain of a common-emitter amplifier is equal to:

a) β

b) gm × Rc

c) ro / (rπ + re)

d) re / (rπ + re)

6. The small signal output impedance is represented by:

a) ro

b) β

c) rπ

d) Rc

7. The rπ component in the small signal model represents:

a) Base-emitter resistance

b) Base-collector resistance

c) Emitter resistance

d) Collector resistance

8. The small signal transconductance (gm) is a measure of the:

a) Amplification of input voltage

b) Amplification of input current

c) Amplification of output voltage

d) Amplification of output current

9. In the small signal model, the coupling capacitor is represented as:

a) A short circuit

b) An open circuit

c) A resistor

d) A capacitor

10. The small signal model assumes that the BJT is operating in which mode?

a) Active mode

b) Saturation mode

c) Cutoff mode

d) Reverse active mode

11. The hybrid-pi model is commonly used for which BJT configuration?

a) Common-base

b) Common-emitter

c) Common-collector

d) All of the above

12. The Miller effect is a phenomenon that affects the:

a) Input impedance of a BJT circuit

b) Output impedance of a BJT circuit

c) Voltage gain of a BJT circuit

d) Current gain of a BJT circuit

13. The small signal model neglects which type of nonlinearity in a BJT?

a) Exponential

b) Quadratic

c) Cubic

d) Harmonic

14. The small signal model is most accurate when the BJT operates in which region?

a) Active region

b) Saturation region

c) Cutoff region

d) All regions

15. The rπ component in the small signal model is approximately equal to:

a) β / gm

b) gm / β

c) β × gm

d) gm × β

16. The small signal model assumes that the AC signal applied to the BJT is:

a) Purely sinusoidal

b) Square wave

c) Pulse waveform

d) Random noise

17. The common-base configuration in the small signal model exhibits:

a) Voltage gain

b) Current gain

c) Power gain

d) Phase shift

18. The early effect in a BJT is represented in the small signal model by:

a) ro

b) rπ

c) gm

d) β

19. The T-model is a simplified small signal model used to analyze the:

a) Common-emitter configuration

b) Common-base configuration

c) Common-collector configuration

d) All configurations

20. The small signal model allows for the analysis of:

a) High-frequency behavior of a BJT circuit

b) Low-frequency behavior of a BJT circuit

c) Transient behavior of a BJT circuit

d) Nonlinear behavior of a BJT circuit

12. Summary

Small-signal models of a BJT are simplified representations used to analyze the behavior of the transistor under small variations around its operating point. The hybrid-π and h-parameter models are commonly used. These models assume linearity and neglect non-linear effects, making analysis easier. Small-signal models help determine parameters like voltage gain, current gain, input/output impedances, and frequency response. They assist in designing and optimizing BJT-based amplifiers for various applications. However, these models have limitations and do not accurately predict distortion or consider certain factors like temperature variations. Nonetheless, they provide valuable insights into BJT behavior, aiding in circuit analysis, stability assessment, and frequency response characterization.

13. Terminal Questions

- What is the purpose of small-signal modeling in BJT analysis?

- What are the key assumptions made in small-signal models of BJTs?

- What are the main differences between the hybrid-π and h-parameter models for BJTs?

- How are the small-signal models derived from the large-signal characteristics of BJTs?

- What are the limitations of small-signal models in representing the behavior of BJTs?

- How is the small-signal voltage gain calculated using the hybrid-π model?

- What is the significance of input and output resistances in the small-signal models?

- How do small-signal models help in determining the frequency response of BJT amplifiers?

- Can small-signal models be used to analyze both voltage-driven and current-driven amplifier configurations?

- What are the parameters that can be determined from the h-parameter model of a BJT?

- How do small-signal models assist in determining the input and output impedances of BJT amplifiers?

- Can small-signal models accurately predict the distortion characteristics of BJT amplifiers?

- How do temperature variations affect the small-signal models of BJTs?

- What are the applications of small-signal models in audio amplifier design?

- How can small-signal models be used to analyze the stability of BJT amplifiers?

- What is the role of coupling capacitors in small-signal modeling of BJT circuits?

- How do small-signal models account for the Early effect in BJTs?

- What are the differences between small-signal and large-signal analysis in BJT circuits?

- How can small-signal models be extended to consider the effects of parasitic elements in BJT circuits?

14. Case Studies (Co Wise)

Case Study: Design of a BJT Amplifier for Audio Applications

Overview: An audio equipment manufacturer is developing a new amplifier for high-quality audio reproduction. The design team decides to use a BJT amplifier configuration due to its low distortion, good linearity, and wide bandwidth characteristics.

15. Answer Key

1. c) Both DC and AC conditions

2. c) Piecewise linear approximation

3. b) β

4. a) rπ

5. b) gm × Rc

6. a) ro

7. a) Base-emitter resistance

8. b) Amplification of input current

9. b) An open circuit

10. a) Active mode

11. b) Common-emitter

12. c) Voltage gain of a BJT circuit

13. a) Exponential

14. a) Active region

15. a) β / gm

16. a) Purely sinusoidal

17. b) Current gain

18. a) ro

19. c) Common-collector configuration

20. a) High-frequency behavior of a BJT circuit

16. Glossary

Small-signal model: A simplified representation of a BJT used to analyze its behavior under small variations or signals around the operating point. It assumes linearity and neglects non-linear effects.

Hybrid-π model: A commonly used small-signal model for BJTs that represents the transistor as a combination of resistors and controlled current and voltage sources. It is suitable for voltage-driven amplifier configurations.

H-parameter model: Another widely used small-signal model for BJTs that represents the transistor as a combination of input and output resistances and current gains. It is often used in current-driven amplifier configurations.

Voltage gain: The amplification factor of a BJT amplifier for small input signals. It is determined using the small-signal models.

Input and output resistances: The resistances seen at the input and output ports of a BJT amplifier when operating in small-signal conditions. These parameters affect the amplifier's overall performance.

Frequency response: The variation in gain and phase of a BJT amplifier as a function of frequency. Small-signal models aid in analyzing and predicting the frequency response characteristics.

Linearity: The ability of a BJT amplifier to faithfully reproduce the input signal without introducing distortion. Small-signal models assume linearity for accurate analysis.

Input and output impedances: The effective resistances seen at the input and output terminals of a BJT amplifier under small-signal conditions. These parameters impact signal transfer and matching with other components.

Temperature variations: Changes in temperature affecting the small-signal models and transistor parameters. Thermal effects can impact BJT behavior.

Distortion: Unwanted changes or alterations introduced to the signal by a BJT amplifier due to non-linear behavior. Small-signal models do not accurately predict distortion.

References of books, sites, links

17. References of books, sites, links

Text Books:

1. Robert L. Boylestad and Louis Nashelsky - “Electronic Devices and Circuit Theory”, 7th Edition, Prentice Hall Publication (1998).

2.Ramakanth A. Gaykwad – “Op-Amps and Linear IC Applications”, 4th Edition, Pearson Education (2015)

3. Thomas L. Floyd – “Electronic Devices – Electron Flow Version”, 4th Edition, Pearson Education (2001)

4. A.S. Sedra&K.C.Smith, Microelectronics Circuits, 7th Edition, Oxford University Press (2014)

Reference Books:

1. Jacob Millman - “Electronic Devices and Circuits”, 4th Edition, McGraw Hill Education (2015)

2. D Roy Choudhury and Shail B. Jain – “Linear Integrated Circuits”, 4th Edition, New Age International Pvt.Ltd. (2017)

3. David A. Bell - “Electronic Devices and Circuits”, 5th Edition, Oxford University Press (2008)

4. Behzad Razavi – “Design of Analog CMOS Integrated Circuit”,2nd Edition, McGraw Hill Education (2017)

5. M. S. Tyagi “Introduction to Semiconductor Materials and Devices”, 2nd Edition, John Wiley & Sons Inc (2008)

Weblinks/MOOCs:

1. https://nptel.ac.in/courses/108102095

2. https://www.coursera.org/learn/electronics?

18. Keywords

Small-signal model, BJT, hybrid-π model, h-parameter model, voltage gain, input resistance, output resistance, frequency response, linearity, impedance, temperature variations, distortion, coupling capacitors, stability, parasitic elements, large-signal analysis, high-frequency analysis, amplifier design, biasing conditions, linear device.