|  | KONERU LAKSHMAIAH EDUCATION FOUNDATION (Deemed to be University estd, u/s, 3 of the UGC Act, 1956) (NAAC Accredited “A++” Grade University) Green Fields, Guntur District, A.P., India – 522502 Department of Electronics and Communication Engineering (DST - FIST Sponsored Department) |  |
| --- | --- | --- |

B.Tech.II ECE PROGRAM

A.Y.2023-24ODD, Semester-III

24EC22104 Analog Electronic Circuit Design

CO1

SESSION-7

Common emitter Amplifier

1.Course Description (Description about the subject)

This course provides the student with the fundamental skills to apply the basics of semiconductor and devices like Bipolar junction transistor, FET, MOSFET and operational amplifier. It would build mathematical and numerical background for design of electronics circuit. Students equipped with the knowledge and skilling provided in the course would enable them to design, develop electronic circuits for real time applications.

2.Aim

The aim of this course is to make the students to apply the basics of semiconductor physics, outline the input and output characteristics of bipolar and unipolar transistors such as JFET and MOSFET, interpret its behavior under different biasing condition, discuss the characteristics of Op-amp, analyze its various linear, nonlinear applications, and characterize the design of an OP-amp filters, Oscillators, and its applications.

3. Instructional Objectives (Course Objectives)

- To impart knowledge on working of a bipolar transistor, its configuration, and its applications.

- To impart knowledge on the need of biasing, types of biasing and interpret its performance.

- To impart knowledge on working of a JFET and MOSFET and interpret its characteristics and parameters.

- To impart knowledge on operational amplifiers and analyse the characteristics of Op-Amp circuits to perform various linear and nonlinear applications.

- To impart knowledge on Operational amplifiers to perform as Active filters and Oscillators and analyse its applications.

- To familiarize with theoretical and mathematical aspects related to electronic circuits, such that student will be able to design electronic systems for real time applications.

4.Learning Outcomes (Course Outcome)

| CO.No. | Course Outcome (CO) | PO/PSO | BTL |
| --- | --- | --- | --- |
| CO 1 | Apply the knowledge of Semiconductor physics and discuss BJT configurations and its applications | PO1, PSO1 | 3 |
| CO 2 | Apply the limitations of BJT and discuss the characteristics and applications of Field Effect Transistors | PO1, PSO1 | 3 |
| CO 3 | Apply the linear and nonlinear circuits approaches and realise the characteristics of operational Amplifiers | PO1, PO2, PSO1 | 4 |
| CO 4 | Apply the concept of feedback system and realise the working principles of Oscillators and multivibrators | PO1, PO2, PSO1 | 4 |
| CO 5 | Design and analyse analog circuits for real time applications using Passive and Active Components. | PO2, PO3, PSO1 | 4 |
| CO 6 | Simulate and analyse Electronic Circuit using Multisim and myDAQ. | PO2, PO3, PSO1 | 4 |

5. Module Description (CO-1 Description)

This module provides the application and analysis the basic concept of BJT and biasing of BJT for amplification of signals

6. Session Outcomes

- Describe the common emitter BJT amplifier

- Demonstrate the applications of CE BJT as amplifier.

7. Session Introduction

The common emitter amplifier is a widely used configuration in bipolar junction transistor (BJT) amplifiers. It provides high voltage gain and good input-output phase relationship. In this configuration, the emitter is common between the input and output, while the collector is the output and the base is the input. The input signal is applied to the base-emitter junction, and the output is taken from the collector-emitter junction. The common emitter amplifier operates in an inverting mode, where the phase of the output signal is opposite to that of the input. It offers versatility in various applications such as audio amplification, signal processing, and voltage amplification, making it a fundamental building block in analog electronic systems.

8. Session description

The common emitter amplifier is a three basic single-stage bipolar junction transistor and is used as a voltage amplifier. The input of this amplifier is taken from the base terminal, the output is collected from the collector terminal and the emitter terminal is common for both the terminals. The basic symbol of the common emitter amplifier is shown below.

Figure 1: common emitter amplifier

8.1 Common Emitter Amplifier Configuration

In electronic circuit design, there are three kinds of transistor configurations are used like common emitter, common base, and common collector, In that, the most frequently used one is common emitter due to its main attributes. This kind of amplifier includes the signal which is given to the base terminal then the output is received from the collector terminal of the circuit. But, as the name suggests, the main attribute of the emitter circuit is familiar for both the input as well as output. The configuration of a common emitter transistor is widely used in most electronic circuit designs. This configuration is evenly appropriate to both the transistors like PNP and NPN transistors but NPN transistors are most frequently used due to the widespread use of these transistors. In Common Emitter Amplifier Configuration, the Emitter of a BJT is common to both the input and output signal as shown below. The arrangement is the same for a PNP transistor, but bias will be opposite w.r.t NPN transistor.

Figure 2: Common Emitter Amplifier Configuration

8.2 Operation of Common Emitter Amplifier

When a signal is applied across the emitter-base junction, the forward bias across this junction increases during the upper half cycle. This leads to an increase in the flow of electrons from the emitter to a collector through the base, hence increases the collector current. The increasing collector current makes more voltage drops across the collector load resistor RC. The negative half cycle decreases the forward bias voltage across the emitter-base junction. The decreasing collector-base voltage decreases the collector current in the whole collector resistor Rc. Thus, the amplified load resistor appears across the collector resistor. The common emitter amplifier circuit is shown above.

8.3 Working of Common Emitter Amplifier

The below circuit diagram shows the working of the common emitter amplifier circuit and it consists of voltage divider biasing, used to supply the base bias voltage as per the necessity. The voltage divider biasing has a potential divider with two resistors are connected in a way that the midpoint is used for supplying base bias voltage.

Figure 3: Working of Common Emitter Amplifier

There are different types of electronic components in the common emitter amplifier which are R1 resistor is used for the forward bias, the R2 resistor is used for the development of bias, the RL resistor is used at the output it is called the load resistance. The RE resistor is used for thermal stability. The C1 capacitor is used to separate the AC signals from the DC biasing voltage and the capacitor is known as the coupling capacitor. The figure shows that the bias vs gain common emitter amplifier transistor characteristics if the R2 resistor increases then there is an increase in the forward bias and R1 & bias are inversely proportional to each other. The alternating current is applied to the base of the transistor of the common emitter amplifier circuit then there is a flow of small base current. Hence there is a large amount of current flow through the collector with the help of the RC resistance. The voltage near the resistance RC will change because the value is very high and the values are from 4 to 10kohm. Hence there is a huge amount of current present in the collector circuit which amplified from the weak signal, therefore common emitter transistors work as an amplifier circuit.

8.4 Voltage Gain of Common Emitter Amplifier

The current gain of the common emitter amplifier is defined as the ratio of change in collector current to the change in base current. The voltage gain is defined as the product of the current gain and the ratio of the output resistance of the collector to the input resistance of the base circuits. The following equations show the mathematical expression of the voltage gain and the current gain.

β = ΔIc/ ΔIb

Av = β Rc/Rb

8.5 Circuit Elements and their Functions

The common emitter amplifier circuit elements and their functions are discussed below.

Biasing Circuit/ Voltage Divider

The resistances R1, R2, and RE used to form the voltage biasing and stabilization circuit. The biasing circuit needs to establish a proper operating Q-point otherwise, a part of the negative half cycle of the signal may be cut-off in the output.

Input Capacitor (C1)

The capacitor C1 is used to couple the signal to the base terminal of the BJT. If it is not there, the signal source resistance, Rs will come across R2, and hence, it will change the bias. C1 allows only the AC signal to flow but isolates the signal source from R2

Emitter Bypass Capacitor (CE)

An Emitter bypass capacitor CE is used parallel with RE to provide a low reactance path to the amplified AC signal. If it is not used, then the amplified AC signal following through RE will cause a voltage drop across it, thereby dropping the output voltage.

Coupling Capacitor (C2)

The coupling capacitor C2 couples one stage of amplification to the next stage. This technique used to isolate the DC bias settings of the two coupled circuits.

CE Amplifier Circuit Currents

Base current iB = IB +ib where,

IB = DC base current when no signal is applied.

ib = AC base when AC signal is applied and iB = total base current.

Collector current iC = IC+ic where,

iC = total collector current.

IC = zero signal collector current.

ic = AC collector current when the AC signal is applied.

Emitter Current iE = IE + ie where,

IE = Zero signal emitter current.

Ie = AC emitter current when AC signal is applied.

iE = total emitter current.

8.6 Common Emitter Amplifier analysis

The first step in AC analysis of Common Emitter amplifier circuit is to draw the AC equivalent circuit by reducing all DC sources to zero and shorting all the capacitors. The below figure shows the AC equivalent circuit.

Figure 4: AC analysis of Common Emitter amplifier

The next step in the AC analysis is to draw an h-parameter circuit by replacing the transistor in the AC equivalent circuit with its h-parameter model. The below figure shows the h-parameter equivalent circuit for the CE circuit.

Figure 5: h-parameter circuit

The typical CE circuit performance is summarised below:

Device input impedance, Zb = hie

Circuit input impedance, Zi = R1 || R2 || Zb

Device output impedance, Zc= 1/hoe

Circuit output impedance, Zo = RC || ZC ≈ RC

Circuit voltage gain, Av = -hfe/hie*(Rc|| RL)

Circuit current gain, AI = hfe. RC. Rb/ (Rc+RL) (Rc+hie)

Circuit power gain, Ap = Av * Ai

8.7 CE Amplifier Frequency Response

The voltage gain of a CE amplifier varies with signal frequency. It is because the reactance of the capacitors in the circuit changes with signal frequency and hence affects the output voltage. The curve drawn between voltage gain and the signal frequency of an amplifier is known as frequency response. The below figure shows the frequency response of a typical CE amplifier.

Figure 6: CE Amplifier Frequency Response

From the above graph, we observe that the voltage gain drops off at low (< FL) and high (> FH) frequencies, whereas it is constant over the mid-frequency range (FL to FH).

At Low Frequencies (< FL) The reactance of coupling capacitor C2 is relatively high and hence very small part of the signal will pass from the amplifier stage to the load.

Moreover, CE cannot shunt the RE effectively because of its large reactance at low frequencies. These two factors cause a drops off of voltage gain at low frequencies.

At High Frequencies (> FH) The reactance of coupling capacitor C2 is very small and it behaves as a short circuit. This increases the loading effect of the amplifier stage and serves to reduce the voltage gain.

Moreover, at high frequencies, the capacitive reactance of base-emitters junction is low which increases the base current. This frequency reduces the current amplification factor β. Due to these two reasons, the voltage gain drops off at a high frequency.
At Mid Frequencies (FL to FH) The voltage gain of the amplifier is constant. The effect of the coupling capacitor C2 in this frequency range is such as to maintain a constant voltage gain. Thus, as the frequency increases in this range, the reactance of CC decreases, which tends to increase the gain.

However, at the same time, lower reactance means higher almost cancel each other, resulting in a uniform fair at mid-frequency.

We can observe the frequency response of any amplifier circuit is the difference in its performance through changes within the input signal’s frequency because it shows the frequency bands where the output remains fairly stable. The circuit bandwidth can be defined as the frequency range either small or big among ƒH & ƒL.

So from this, we can decide the voltage gain for any sinusoidal input in a given range of frequency. The frequency response of a logarithmic presentation is the Bode diagram. Most of the audio amplifiers have a flat frequency response that ranges from 20 Hz – 20 kHz. For an audio amplifier, the frequency range is known as Bandwidth.

Frequency points like ƒL & ƒH are related to the lower corner & the upper corner of the amplifier which are the gain falls of the circuits at high as well as low frequencies. These frequency points are also known as decibel points. So the BW can be defined as

BW = fH – fL

The dB (decibel) is 1/10th of a B (bel), is a familiar non-linear unit to measure gain & is defined like 20log10(A). Here ‘A’ is the decimal gain which is plotted over the y-axis.

The maximum output can be obtained through the zero decibels which communicate toward a magnitude function of unity otherwise it occurs once Vout = Vin when there is no reduction at this frequency level, so

VOUT/VIN = 1, so 20log(1) = 0dB

We can notice from the above graph, the output at the two cut-off frequency points will decrease from 0dB to -3dB & continues to drop at a fixed rate. This reduction within gain is known commonly as the roll-off section of the frequency response curve. In all basic filter and amplifier circuits, this roll-off rate can be defined as 20dB/decade, which is equal to a 6dB/octave rate. So, the order of the circuit is multiplied with these values.

These -3dB cut-off frequency points will describe the frequency where the o/p gain can be decreased to 70 % of its utmost value. After that, we can properly say that the frequency point is also the frequency at which the gain of the system has reduced to 0.7 of its utmost value.

9. Activities/ Case studies/related to the session

Activities:

- For the circuit shown in Fig. 4, determine (i) the current gain (ii) the voltage gain and (iii) the power gain. Neglect the a.c. emitter resistance for the transistor.

- For the circuit shown in Fig. 5, find (i) the d.c. bias levels (ii) d.c. voltages across the capacitors (iii) a.c. emitter resistance (iv) voltage gain and (v) state of the transistor.

10. Examples & contemporary extracts of articles/ practices to convey the idea of the session

Common Emitter Amplifier Circuit Working & Its Characteristics (Source: https://www.elprocus.com)

Extract:  The common emitter amplifier is a three basic single-stage bipolar junction transistor and is used as a voltage amplifier. The input of this amplifier is taken from the base terminal, the output is collected from the collector terminal and the emitter terminal is common for both the terminals.

11. SAQ's-Self Assessment Questions

Certainly! Here are 20 multiple-choice questions (MCQs) on the BJT common emitter amplifier:

1. The common emitter amplifier is a type of BJT amplifier that has the input applied to the:

a) Base terminal

b) Emitter terminal

c) Collector terminal

d) Base-emitter junction

2. In the common emitter amplifier, the output is taken from the:

a) Base terminal

b) Emitter terminal

c) Collector terminal

d) Base-emitter junction

3. The common emitter amplifier provides:

a) Voltage amplification

b) Current amplification

c) Power amplification

d) Both voltage and current amplification

4. The common emitter amplifier exhibits which type of phase relationship between the input and output signals?

a) 0 degrees

b) 90 degrees

c) 180 degrees

d) 360 degrees

5. The common emitter amplifier has a phase inversion property, meaning that the output signal is:

a) In phase with the input signal

b) 180 degrees out of phase with the input signal

c) 90 degrees out of phase with the input signal

d) 360 degrees out of phase with the input signal

6. The voltage gain of the common emitter amplifier is primarily determined by the ratio of:

a) Input impedance to output impedance

b) Base current to collector current

c) Collector current to base current

d) Collector resistance to emitter resistance

7. The common emitter amplifier has a relatively high:

a) Input impedance and low output impedance

b) Output impedance and low input impedance

c) Input and output impedances

d) None of the above

8. The emitter resistor in the common emitter amplifier serves to:

a) Provide voltage gain

b) Provide current gain

c) Stabilize the DC operating point

d) Control the frequency response

9. The coupling capacitor in the common emitter amplifier is used to:

a) Block DC voltage from the input signal

b) Amplify the AC component of the input signal

c) Provide biasing for the transistor

d) Control the gain of the amplifier

10. The bypass capacitor in the common emitter amplifier is connected in parallel with the:

a) Collector resistor

b) Emitter resistor

c) Base resistor

d) Load resistor

11. The purpose of the bypass capacitor in the common emitter amplifier is to:

a) Block DC voltage from the output signal

b) Provide AC coupling between stages

c) Improve the stability of the amplifier

d) Increase the gain of the amplifier

12. The quiescent point (Q-point) in the common emitter amplifier represents:

a) The point of maximum power dissipation

b) The center frequency of operation

c) The DC bias point of the transistor

d) The point of saturation of the transistor

13. The DC operating point of the common emitter amplifier is determined by the:

a) Collector current (IC)

b) Base current (IB)

c) Voltage gain (Av)

d) Power supply voltage (VCC)

14. The load resistor in the common emitter amplifier is connected to the:

a) Base terminal

b) Emitter terminal

c) Collector terminal

d) Ground terminal

15. The load resistor in the common emitter amplifier is responsible for:

a) Amplifying the input signal

b) Providing biasing for the transistor

c) Setting the DC operating point

d) Impedance matching between stages

16. The common emitter amplifier operates in which region of the BJT's characteristic curve?

a) Active region

b) Saturation region

c) Cutoff region

d) Reverse active region

17. The AC coupling capacitor in the common emitter amplifier

allows for the coupling of:

a) DC signals between stages

b) AC signals between stages

c) Both DC and AC signals between stages

d) None of the above

18. The Miller effect in the common emitter amplifier refers to the:

a) Capacitive coupling between the input and output

b) Amplification of the input signal

c) Phase inversion between the input and output

d) Increase in effective input capacitance

19. The common emitter amplifier exhibits a relatively:

a) High input impedance and low output impedance

b) Low input impedance and high output impedance

c) Balanced input and output impedances

d) None of the above

20. The common emitter amplifier is commonly used in applications such as:

a) Audio amplifiers

b) RF amplifiers

c) Voltage regulators

d) All of the above

12. Summary

The common emitter amplifier is a widely used configuration in bipolar junction transistor (BJT) amplifiers. It provides high voltage gain and operates in an inverting mode. The input signal is applied to the base-emitter junction, and the output is taken from the collector-emitter junction. The common emitter amplifier offers versatility in applications such as audio amplification and voltage amplification. It utilizes biasing to establish the proper operating point and employs coupling and bypass capacitors to block DC bias and allow AC signals to pass. The amplifier's performance can be affected by factors like temperature variations, component values, and stability considerations. Despite its limitations, the common emitter amplifier serves as a fundamental building block in analog electronic systems, delivering amplification and signal processing capabilities.

13. Terminal Questions

- What is a common emitter amplifier, and what is its configuration?

- How does a common emitter amplifier provide voltage gain?

- What are the input and output terminals of a common emitter amplifier?

- How does the common emitter amplifier achieve signal inversion?

- What is the purpose of biasing in a common emitter amplifier?

- What is the role of the coupling capacitor in a common emitter amplifier?

- How does the bypass capacitor affect the performance of a common emitter amplifier?

- What is the impact of emitter degeneration resistor on the common emitter amplifier?

- How is the voltage gain calculated for a common emitter amplifier?

- What are the input and output impedances of a common emitter amplifier?

- What is the effect of temperature variations on the performance of a common emitter amplifier?

- How does changing the values of resistors and capacitors impact the gain and frequency response of a common emitter amplifier?

- What are the advantages of using a common emitter amplifier in signal amplification?

- How does negative feedback affect the performance of a common emitter amplifier?

- Can a common emitter amplifier be used for both AC and DC signal amplification?

- What are the limitations of a common emitter amplifier?

- How can stability issues be addressed in a common emitter amplifier design?

- What is the role of coupling transformers in common emitter amplifiers?

- How does the choice of BJT transistor affect the performance of a common emitter amplifier?

- Can a common emitter amplifier be used as a power amplifier? If so, what considerations should be taken into account?

14. Case Studies (Co Wise)

Case Study: Design of a BJT Amplifier for Audio Applications

Overview: An audio equipment manufacturer is developing a new amplifier for high-quality audio reproduction. The design team decides to use a BJT amplifier configuration due to its low distortion, good linearity, and wide bandwidth characteristics.

15. Answer Key

1. a) Base terminal

2. c) Collector terminal

3. d) Both voltage and current amplification

4. c) 180 degrees

5. b) 180 degrees out of phase with the input signal

6. c) Collector current to base current

7. a) Input impedance and low output impedance

8. c) Stabilize the DC operating point

9. a) Block DC voltage from the input signal

10. b) Emitter resistor

11. a) Block DC voltage from the output signal

12. c) The DC bias point of the transistor

13. a) Collector current (IC)

14. c) Collector terminal

15. d) Impedance matching between stages

16. a) Active region

17. b) AC signals between stages

18. d) Increase in effective input capacitance

19. a) High input impedance and low output impedance

20. d) All of the above

16. Glossary

Bipolar Junction Transistor (BJT): A three-terminal electronic device that consists of two p-n junctions. It can operate as an amplifier or a switch.

Common Emitter (CE) Configuration: A configuration in which the emitter terminal of a BJT is common to both the input and output circuits, while the base is the input and the collector is the output.

Collector Current (IC): The current flowing into the collector terminal of a BJT in the common emitter configuration.

Base Current (IB): The current flowing into the base terminal of a BJT in the common emitter configuration.

Emitter Current (IE): The sum of the collector and base currents in a BJT, given by the equation IE = IC + IB.

17. References of books, sites, links

Text Books:

1. Robert L. Boylestad and Louis Nashelsky - “Electronic Devices and Circuit Theory”, 7th Edition, Prentice Hall Publication (1998).

2.Ramakanth A. Gaykwad – “Op-Amps and Linear IC Applications”, 4th Edition, Pearson Education (2015)

3. Thomas L. Floyd – “Electronic Devices – Electron Flow Version”, 4th Edition, Pearson Education (2001)

4. A.S. Sedra&K.C.Smith, Microelectronics Circuits, 7th Edition, Oxford University Press (2014)

Reference Books:

1. Jacob Millman - “Electronic Devices and Circuits”, 4th Edition, McGraw Hill Education (2015)

2. D Roy Choudhury and Shail B. Jain – “Linear Integrated Circuits”, 4th Edition, New Age International Pvt.Ltd. (2017)

3. David A. Bell - “Electronic Devices and Circuits”, 5th Edition, Oxford University Press (2008)

4. Behzad Razavi – “Design of Analog CMOS Integrated Circuit”,2nd Edition, McGraw Hill Education (2017)

5. M. S. Tyagi “Introduction to Semiconductor Materials and Devices”, 2nd Edition, John Wiley & Sons Inc (2008)

Weblinks/MOOCs:

1. https://nptel.ac.in/courses/108102095

2. https://www.coursera.org/learn/electronics?

18. Keywords

Common emitter amplifier, BJT, configuration, voltage gain, inverting, input, output, biasing, coupling capacitor, bypass capacitor, emitter degeneration resistor, voltage gain calculation, input impedance, output impedance, temperature variations, component values, stability, negative feedback, AC signal amplification, limitations, versatility, audio amplification, voltage amplification, coupling transformer, power amplifier.