|  | KONERU LAKSHMAIAH EDUCATION FOUNDATION (Deemed to be University estd, u/s, 3 of the UGC Act, 1956) (NAAC Accredited “A++” Grade University) Green Fields, Guntur District, A.P., India – 522502 Department of Electronics and Communication Engineering (DST - FIST Sponsored Department) |  |
| --- | --- | --- |

B.Tech.II ECE PROGRAM

A.Y.2023-24ODD, Semester-III

24EC22104 Analog Electronic Circuit Design

CO1

SESSION-8

Concept of feedback in BJT amplifiers

1.Course Description (Description about the subject)

This course provides the student with the fundamental skills to apply the basics of semiconductor and devices like Bipolar junction transistor, FET, MOSFET and operational amplifier. It would build mathematical and numerical background for design of electronics circuit. Students equipped with the knowledge and skilling provided in the course would enable them to design, develop electronic circuits for real time applications.

2.Aim

The aim of this course is to make the students to apply the basics of semiconductor physics, outline the input and output characteristics of bipolar and unipolar transistors such as JFET and MOSFET, interpret its behavior under different biasing condition, discuss the characteristics of Op-amp, analyze its various linear, nonlinear applications, and characterize the design of an OP-amp filters, Oscillators, and its applications.

3. Instructional Objectives (Course Objectives)

- To impart knowledge on working of a bipolar transistor, its configuration, and its applications.

- To impart knowledge on the need of biasing, types of biasing and interpret its performance.

- To impart knowledge on working of a JFET and MOSFET and interpret its characteristics and parameters.

- To impart knowledge on operational amplifiers and analyse the characteristics of Op-Amp circuits to perform various linear and nonlinear applications.

- To impart knowledge on Operational amplifiers to perform as Active filters and Oscillators and analyse its applications.

- To familiarize with theoretical and mathematical aspects related to electronic circuits, such that student will be able to design electronic systems for real time applications.

4.Learning Outcomes (Course Outcome)

| CO.No. | Course Outcome (CO) | PO/PSO | BTL |
| --- | --- | --- | --- |
| CO 1 | Apply the knowledge of Semiconductor physics and discuss BJT configurations and its applications | PO1, PSO1 | 3 |
| CO 2 | Apply the limitations of BJT and discuss the characteristics and applications of Field Effect Transistors | PO1, PSO1 | 3 |
| CO 3 | Apply the linear and nonlinear circuits approaches and realise the characteristics of operational Amplifiers | PO1, PO2, PSO1 | 4 |
| CO 4 | Apply the concept of feedback system and realise the working principles of Oscillators and multivibrators | PO1, PO2, PSO1 | 4 |
| CO 5 | Design and analyse analog circuits for real time applications using Passive and Active Components. | PO2, PO3, PSO1 | 4 |
| CO 6 | Simulate and analyse Electronic Circuit using Multisim and myDAQ. | PO2, PO3, PSO1 | 4 |

5. Module Description (CO-1 Description)

This module provides the application and analysis the basic concept of BJT and biasing of BJT for amplification of signals

6. Session Outcomes

- Describe the feedback conditions in Amplifiers.

- Demonstrate the significance of feedback in BJT amplifiers.

7. Session Introduction

Feedback in BJT amplifier circuits refers to the technique of returning a portion of the output signal back to the input of the amplifier. It plays a crucial role in modifying and controlling the amplifier's performance. Feedback can be positive or negative, depending on whether the returned signal reinforces or opposes the input signal. Negative feedback is the most commonly used form of feedback in BJT amplifier circuits. It involves feeding a fraction of the output signal back to the input with the opposite polarity. The purpose of negative feedback is to stabilize the amplifier, improve linearity, reduce distortion, and enhance overall performance. It helps in achieving desired gain, impedance matching, and frequency response characteristics. Positive feedback, on the other hand, amplifies and reinforces the input signal. It is typically used in specific applications like oscillators or in certain signal processing circuits where intentional signal regeneration is required. Feedback in BJT amplifier circuits can be implemented using various techniques, such as voltage feedback, current feedback, or impedance feedback. The choice of feedback technique depends on the specific requirements of the amplifier design. Feedback is a powerful tool in BJT amplifier circuits, allowing engineers to tailor the amplifier's behavior, improve performance, and achieve desired characteristics for a wide range of applications.

8. Session description

If some percentage of an amplifier’s output signal is connected to the input, so that the amplifier amplifies part of its output signal, we have what is known as feedback.

8.1 Feedback Categories

Feedback comes in two varieties: positive (also called regenerative), and negative (also called degenerative).

8.2 Positive feedback

Reinforces the direction of an amplifier’s output voltage change, while negative feedback does just the opposite.

A familiar example of feedback happens in public-address (“PA”) systems where someone holds the microphone too close to a speaker: a high-pitched “whine” or “howl” ensues, because the audio amplifier system is detecting and amplifying its noise. Specifically, this is an example of positive or regenerative feedback, as any sound detected by the microphone is amplified and turned into a louder sound by the speaker, which is then detected by the microphone again, and so on . . . the result being a noise of steadily increasing volume until the system becomes “saturated” and cannot produce any more volume. One might wonder what possible benefit feedback is to an amplifier circuit, given such an annoying example as PA system “howl.” If we introduce positive, or regenerative, feedback into an amplifier circuit, it tends to create and sustain oscillations, the frequency of which is determined by the values of components handling the feedback signal from output to input. This is one way to make an oscillator circuit to produce AC from a DC power supply. Oscillators are very useful circuits, and so feedback has a definite, practical application for us.

8.3 Negative feedback

On the other hand, Negative feedback has a “dampening” effect on an amplifier: if the output signal happens to increase in magnitude, the feedback signal introduces a decreasing influence into the input of the amplifier, thus opposing the change in the output signal. While positive feedback drives an amplifier circuit toward a point of instability (oscillations), negative feedback drives it the opposite direction: toward a point of stability.

An amplifier circuit equipped with some amount of negative feedback is not only more stable, but it distorts the input waveform less and is generally capable of amplifying a wider range of frequencies. The tradeoff for these advantages (there just has to be a disadvantage to negative feedback, right?) is decreased gain. If a portion of an amplifier’s output signal is “fed back” to the input to oppose any changes in the output, it will require a greater input signal amplitude to drive the amplifier’s output to the same amplitude as before. This constitutes a decreased gain. However, the advantages of stability, lower distortion, and greater bandwidth are worth the tradeoff in reduced gain for many applications. Let’s examine a simple amplifier circuit and see how we might introduce negative feedback into it, starting with the figure below.

Figure 1: Feedback categories

8.4 Common-emitter amplifier without feedback.

The amplifier configuration shown here is a common-emitter, with a resistor bias network formed by R1and R2. The capacitor couples Vinput to the amplifier so that the signal source doesn’t have a DC voltage imposed on it by the R1/R2 divider network. Resistor R3 serves the purpose of controlling voltage gain. We could omit it for maximum voltage gain, but since base resistors like this are common in common-emitter amplifier circuits, we’ll keep it in this schematic. Like all common-emitter amplifiers, this one inverts the input signal as it is amplified. In other words, a positive-going input voltage causes the output voltage to decrease, or move toward the negative, and vice versa. The oscilloscope waveforms are shown in the figure below.

Figure 2: Common-emitter amplifier without feedback

8.5 Common-emitter amplifier, no feedback, with reference waveforms for comparison.

Because the output is inverted, or mirror-image, reproduction of the input signal, any connection between the output (collector) wire and the input (base) wire of the transistor in the figure below will result in negative feedback.

Figure 3: CE amplifier with reference waveforms

8.6 Negative feedback, collector feedback, decreases the output signal.

The resistances of R1, R2, R3, and Rfeedback function together as a signal-mixing network so that the voltage seen at the base of the transistor (with respect to ground) is a weighted average of the input voltage and the feedback voltage, resulting in a signal of reduced amplitude going into the transistor. So, the amplifier circuit in the figure above will have reduced voltage gain, but improved linearity (reduced distortion) and increased bandwidth.

A resistor connecting the collector to base is not the only way to introduce negative feedback into this amplifier circuit, though. Another method, although more difficult to understand at first, involves the placement of a resistor between the transistor’s emitter terminal and circuit ground in the figure below.

Figure 4: CE amplifier with reference waveforms

8.7 Emitter feedback: A different method of introducing negative feedback into a circuit.

This new feedback resistor drops voltage proportional to the emitter current through the transistor, and it does so in such a way as to oppose the input signal influence on the base-emitter junction of the transistor. Let’s take a closer look at the emitter-base junction and see what difference this new resistor makes in the figure below.

With no feedback resistor connecting the emitter to ground in the figure below (a), whatever level of input signal (Vinput) makes it through the coupling capacitor and R1/R2/R3 resistor network will be impressed directly across the base-emitter junction as the transistor’s input voltage (VB-E). In other words, with no feedback resistor, VB-E equals Vinput. Therefore, if Vinput increases by 100 mV, then VB-E increases by 100 mV: a change in one is the same as a change in the other since the two voltages are equal to each other. Now let’s consider the effects of inserting a resistor (Rfeedback) between the transistor’s emitter lead and ground in Figure below (b).

Figure 5: Emitter feedback

(a) No feedback vs (b) emitter feedback. A waveform at the collector is inverted with respect to the base. At (b) the emitter waveform is in-phase (emitter follower) with base, out of phase with collector. Therefore, the emitter signal subtracts from the collector output signal.

Note how the voltage dropped across Rfeedback adds with VB-E to equal Vinput. With Rfeedback in the Vinput—VB-E loop, VB-E will no longer be equal to Vinput. We know that Rfeedback will drop a voltage proportional to emitter current, which is in turn controlled by the base current, which is in turn controlled by the voltage dropped across the base-emitter junction of the transistor (VB-E). Thus, if Vinput were to increase in a positive direction, it would increase VB-E, causing more base current, causing more collector (load) current, causing more emitter current, and causing more feedback voltage to be dropped across Rfeedback. This increase of voltage drop across the feedback resistor, though, subtracts from Vinput to reduce the VB-E, so that the actual voltage increase for VB-E will be less than the voltage increase of Vinput. No longer will a 100 mV increase in Vinput result in a full 100 mV increase for VB-E, because the two voltages are not equal to each other.

Consequently, the input voltage has less control over the transistor than before, and the voltage gain for the amplifier is reduced: just what we expected from negative feedback.

In practical common-emitter circuits, negative feedback isn’t just a luxury; its a necessity for stable operation. In a perfect world, we could build and operate a common-emitter transistor amplifier with no negative feedback, and have the full amplitude of Vinput impressed across the transistor’s base-emitter junction. This would give us a large voltage gain. Unfortunately, though, the relationship between base-emitter voltage and base-emitter current changes with temperature, as predicted by the “diode equation.” As the transistor heats up, there will be less of a forward voltage drop across the base-emitter junction for any given current. This causes a problem for us, as the R1/R2 voltage divider network is designed to provide the correct quiescent current through the base of the transistor so that it will operate in whatever class of operation we desire (in this example, I’ve shown the amplifier working in class-A mode). If the transistor’s voltage/current relationship changes with temperature, the amount of DC bias voltage necessary for the desired class of operation will change. A hot transistor will draw more bias current for the same amount of bias voltage, making it heat up even more, drawing even more bias current. The result, if unchecked, is called thermal runaway.

9. Activities/ Case studies/related to the session

Activities:

1. When negative voltage feedback is applied to an amplifier of gain 100, the overall gain falls to 50.

(i) Calculate the fraction of the output voltage feedback.

(ii) If this fraction is maintained, calculate the value of the amplifier gain required if the overall stage gain is to be 75.

2. With a negative voltage feedback, an amplifier gives an output of 10 V with an input of 0.5 V. When feedback is removed, it requires 0.25 V input for the same output. Calculate (i) gain without feedback (ii) feedback fraction mv.

10. Examples & contemporary extracts of articles/ practices to convey the idea of the session

Feedback Amplifier (Source: www.sciencedirect.com)

Extract: A feedback amplifier is an amplifier having a feedback way that exists between output to input. Like all other amplifiers, feedback amplifiers increase the strength of a signal. But in this amplifier, the ratio of the feedback signal and the input signal works as a feedback factor that measures the sum of feedback. There are two parts to a feedback amplifier: one is an amplifier, and the other is a feedback circuit.

11. SAQ's-Self Assessment Questions

Certainly! Here are 20 multiple-choice questions (MCQs) on feedback in BJT amplifiers:

1. Feedback in BJT amplifiers involves:

a) Diverting a portion of the input signal to the output

b) Diverting a portion of the output signal to the input

c) Increasing the gain of the amplifier

d) Decreasing the stability of the amplifier

2. Negative feedback in BJT amplifiers:

a) Increases the input impedance

b) Increases the output impedance

c) Reduces distortion and improves stability

d) Increases distortion and decreases stability

3. Positive feedback in BJT amplifiers:

a) Increases the gain of the amplifier

b) Reduces the gain of the amplifier

c) Increases distortion and decreases stability

d) Reduces distortion and improves stability

4. The purpose of feedback in BJT amplifiers is to:

a) Increase the power output

b) Decrease the power consumption

c) Improve the linearity and stability

d) Reduce the bandwidth

5. Voltage feedback in BJT amplifiers is achieved by:

a) Connecting a resistor from the output to the input

b) Connecting a capacitor from the output to the input

c) Connecting a resistor from the input to the output

d) Connecting a capacitor from the input to the output

6. Current feedback in BJT amplifiers is achieved by:

a) Connecting a resistor from the output to the input

b) Connecting a capacitor from the output to the input

c) Connecting a resistor from the input to the output

d) Connecting a capacitor from the input to the output

7. The main advantage of negative feedback in BJT amplifiers is:

a) Increased gain

b) Increased distortion

c) Improved linearity and stability

d) Decreased bandwidth

8. The main disadvantage of positive feedback in BJT amplifiers is:

a) Decreased gain

b) Decreased distortion

c) Decreased stability

d) Increased bandwidth

9. The feedback factor is defined as:

a) The ratio of output voltage to input voltage

b) The ratio of output current to input current

c) The ratio of feedback signal to input signal

d) The ratio of feedback signal to output signal

10. The gain of an amplifier with feedback is given by:

a) A = Av × (1 + β)

b) A = Av / (1 + β)

c) A = Av × β

d) A = Av / β

11. In BJT amplifiers, the most commonly used type of feedback is:

a) Voltage feedback

b) Current feedback

c) Positive feedback

d) Negative feedback

12. The Barkhausen stability criterion states that for oscillation to occur, the total phase shift around the loop must be:

a) 0 degrees

b) 90 degrees

c) 180 degrees

d) 360 degrees

13. The Miller effect in BJT amplifiers refers to:

a) The amplification of the input signal

b) The amplification of the output signal

c) The increase in effective input capacitance

d) The decrease in effective output impedance

14. The transfer function of an amplifier with feedback is determined by the:

a) Feedback factor

b) Input signal amplitude

c) Output signal amplitude

d) Power supply voltage

15. Feedback is commonly used in BJT amplifiers to:

a) Increase the power output

b) Reduce the power consumption

c) Control the gain and improve stability

d) Increase the bandwidth

16. The Nyquist stability criterion is used to determine the stability of an amplifier by analyzing the:

a) Gain and phase margins

b) Feedback factor

c) Open-loop gain

d) Closed-loop gain

17. The loop gain in a feedback amplifier is defined as:

a) The gain of the amplifier without feedback

b) The product of the forward gain and feedback factor

c) The ratio of input to output signal

d) The ratio of feedback signal to input signal

18. The dominant pole in a feedback amplifier refers to:

a) The pole with the highest frequency response

b) The pole with the lowest frequency response

c) The pole with the highest gain

d) The pole with the lowest gain

19. The stability of a feedback amplifier is determined by:

a) The gain and phase margins

b) The feedback factor

c) The input impedance

d) The output impedance

20. The purpose of compensation in a feedback amplifier is to:

a) Increase the gain

b) Decrease the gain

c) Improve the stability

d) Reduce the bandwidth

12. Summary

Feedback in BJT amplifier circuits involves returning a portion of the output signal back to the input. Negative feedback, the most common type, improves stability, linearity, and reduces distortion. It can be implemented through voltage, current, or impedance feedback. Negative feedback helps to achieve desired gain, impedance matching, and frequency response characteristics. Positive feedback, less commonly used, amplifies the input signal. Feedback can stabilize amplifier circuits, enhance performance, and compensate for non-idealities. It affects gain, noise performance, and efficiency. Analyzing and measuring the effects of feedback are essential for designing and optimizing BJT amplifier circuits. Overall, feedback is a powerful technique that shapes the behavior and characteristics of BJT amplifiers.

13. Terminal Questions

- What is feedback in the context of BJT amplifier circuits?

- What are the two main types of feedback used in BJT amplifier circuits?

- How does negative feedback affect the performance of a BJT amplifier?

- What are the advantages of using negative feedback in amplifier circuits?

- How does positive feedback differ from negative feedback in terms of its impact on amplifier performance?

- What are the potential drawbacks or challenges of using positive feedback in BJT amplifier circuits?

- How is feedback implemented in BJT amplifier circuits?

- What are the different techniques for implementing feedback in BJT amplifiers?

- How does feedback affect the gain of a BJT amplifier?

- What is the role of feedback in stabilizing amplifier circuits?

- How does feedback impact the linearity and distortion characteristics of a BJT amplifier?

- What is the relationship between feedback and the input/output impedance of an amplifier?

- Can feedback be used to improve the frequency response of a BJT amplifier? If so, how?

- How does the choice of feedback configuration (voltage, current, or impedance) affect amplifier performance?

- How does feedback affect the noise performance of a BJT amplifier?

- Can feedback be used to improve the overall efficiency of a BJT amplifier?

- What are the considerations for implementing feedback in high-frequency BJT amplifier circuits?

- How does feedback impact the stability and oscillation behavior of an amplifier circuit?

- Can feedback be used to compensate for non-idealities in BJT amplifiers?

- How can the effects of feedback be analyzed and measured in BJT amplifier circuits?

14. Case Studies (Co Wise)

Case Study: Design of a BJT Amplifier for Audio Applications

Overview: An audio equipment manufacturer is developing a new amplifier for high-quality audio reproduction. The design team decides to use a BJT amplifier configuration due to its low distortion, good linearity, and wide bandwidth characteristics.

15. Answer Key

1. b) Diverting a portion of the output signal to the input

2. c) Reduces distortion and improves stability

3. c) Increases distortion and decreases stability

4. c) Improve the linearity and stability

5. a) Connecting a resistor from the output to the input

6. c) Connecting a resistor from the input to the output

7. c) Improved linearity and stability

8. c) Decreased stability

9. c) The ratio of feedback signal to input signal

10. b) A = Av / (1 + β)

11. a) Voltage feedback

12. c) 180 degrees

13. c) The increase in effective input capacitance

14. a) Feedback factor

15. c) Control the gain and improve stability

16. a) Gain and phase margins

17. b) The product of the forward gain and feedback factor

18. b) The pole with the lowest frequency response

19. a) The gain and phase margins

20. c) Improve the stability

16. Glossary

Feedback: The process of feeding a portion of the output signal back to the input of an amplifier circuit.

Negative Feedback: Feedback in which the phase of the feedback signal is opposite to the input signal, resulting in reduced distortion, increased stability, and improved linearity.

Positive Feedback: Feedback in which the phase of the feedback signal is the same as the input signal, leading to amplification and potential instability.

Feedback Network: The network or circuitry used to provide the feedback signal in an amplifier circuit. It can be a resistor-capacitor network, an operational amplifier, or other components.

Voltage Feedback: Feedback in which the output voltage is used to generate the feedback signal. It is commonly used in voltage amplifiers to control the gain and improve stability.

Current Feedback: Feedback in which the output current is used to generate the feedback signal. It is often used in current amplifiers to control the input impedance and enhance linearity.

17. References of books, sites, links

Text Books:

1. Robert L. Boylestad and Louis Nashelsky - “Electronic Devices and Circuit Theory”, 7th Edition, Prentice Hall Publication (1998).

2.Ramakanth A. Gaykwad – “Op-Amps and Linear IC Applications”, 4th Edition, Pearson Education (2015)

3. Thomas L. Floyd – “Electronic Devices – Electron Flow Version”, 4th Edition, Pearson Education (2001)

4. A.S. Sedra&K.C.Smith, Microelectronics Circuits, 7th Edition, Oxford University Press (2014)

Reference Books:

1. Jacob Millman - “Electronic Devices and Circuits”, 4th Edition, McGraw Hill Education (2015)

2. D Roy Choudhury and Shail B. Jain – “Linear Integrated Circuits”, 4th Edition, New Age International Pvt.Ltd. (2017)

3. David A. Bell - “Electronic Devices and Circuits”, 5th Edition, Oxford University Press (2008)

4. Behzad Razavi – “Design of Analog CMOS Integrated Circuit”,2nd Edition, McGraw Hill Education (2017)

5. M. S. Tyagi “Introduction to Semiconductor Materials and Devices”, 2nd Edition, John Wiley & Sons Inc (2008)

Weblinks/MOOCs:

1. https://nptel.ac.in/courses/108102095

2. https://www.coursera.org/learn/electronics?

18. Keywords

Feedback, negative feedback, positive feedback, voltage feedback, current feedback, impedance feedback, stability