Signals & Communication Systems -Lab-3

Manipulation / Operations of Continuous Time signals

1.Consider a signal shown in figure:

Determine analytically the following signals.

(𝑎) 𝑥(𝑡 − 1) (𝑏) 𝑥(𝑡 + 1)

(𝑐) 𝑥(2𝑡) (𝑑) 𝑥(−𝑡)

Then develop Matlab scripts and plot them.

Sol:

% Define the time vector

t = -3:0.001:3;

% Define unit step function

u = @(t) double(t >= 0); % Returns 1 where t >= 0, else 0

x = @(t) u(t);           % x(t) is the unit step

% Define transformed signals

y1 = x(t - 1);   % x(t - 1): delayed by 1

y2 = x(t + 1);   % x(t + 1): advanced by 1

y3 = x(2*t);     % x(2t): time-compressed

y4 = x(-t);      % x(-t): time-reversed

% Plotting

figure

% x(t - 1)

subplot(2,2,1);

plot(t, zeros(size(t)), 'k', 'LineWidth', 1); hold on;

plot(zeros(size(t)), t, 'k', 'LineWidth', 1); hold on;

plot(t, y1, 'r', 'LineWidth', 2);

title('x(t - 1)');

axis([-3 3 -0.1 1.5]); axis square;

% x(t + 1)

subplot(2,2,2);

plot(t, zeros(size(t)), 'k', 'LineWidth', 1); hold on;

plot(zeros(size(t)), t, 'k', 'LineWidth', 1); hold on;

plot(t, y2, 'b', 'LineWidth', 2);

title('x(t + 1)');

axis([-3 3 -0.1 1.5]); axis square;

% x(2t)

subplot(2,2,3);

plot(t, zeros(size(t)), 'k', 'LineWidth', 1); hold on;

plot(zeros(size(t)), t, 'k', 'LineWidth', 1); hold on;

plot(t, y3, 'm', 'LineWidth', 2);

title('x(2t)');

axis([-3 3 -0.1 1.5]); axis square;

% x(-t)

subplot(2,2,4);

plot(t, zeros(size(t)), 'k', 'LineWidth', 1); hold on;

plot(zeros(size(t)), t, 'k', 'LineWidth', 1); hold on;

plot(t, y4, 'g', 'LineWidth', 2);

title('x(-t)');

axis([-3 3 -0.1 1.5]); axis square;

Figure: