Sol:

clc; clear; close all;

%% Sampling parameters

Fs = 200;               % Sampling frequency (Hz) - high enough (> 10 Hz)

T = 1/Fs;               % Sampling interval

L = 2048;               % Number of samples

t = (-L/2:L/2-1)*T;     % Symmetric time axis

f = (-L/2:L/2-1)*(Fs/L);% Frequency axis in Hz

%% -------- Signal (a): x(t) = e^{-|t|} cos(10t) --------

x1 = exp(-abs(t)) .* cos(10*t);   % Correct formula with decay

% Fourier Transform using FFT

X1 = fftshift(fft(x1, L));

X1mag = abs(X1)/L;

X1phase = angle(X1);

%% -------- Plot Time-domain --------

figure;

plot(t, x1, 'b', 'LineWidth', 1.5);

xlabel('Time (s)'); ylabel('x_1(t)');

title('Signal x_1(t) = e^{-|t|} cos(10t)');

grid on;

%% -------- Plot Magnitude Spectrum --------

figure;

plot(2*pi*f, X1mag, 'r', 'LineWidth', 1.5); % frequency in rad/s

xlabel('Frequency (rad/s)'); ylabel('|X_1(f)|');

title('Magnitude Spectrum of x_1(t)');

grid on;

%% -------- Plot Phase Spectrum --------

figure;

plot(2*pi*f, unwrap(X1phase), 'k', 'LineWidth', 1.5);

xlabel('Frequency (rad/s)'); ylabel('Phase (rad)');

title('Phase Spectrum of x_1(t)');

grid on;

Figure:

BBb

B     Sol: clc; clear; close all;

%% Sampling parameters

Fs = 50000;              % Sampling frequency (50 kHz, > 2*5500 to satisfy Nyquist)

T = 1/Fs;                % Sampling interval

L = 8192;                % Number of samples

t = (0:L-1)*T;           % Time vector

f = (-L/2:L/2-1)*(Fs/L); % Frequency axis

%% -------- Signal (b): x(t) = 2cos(2π500t)cos(2π5000t) --------

x2 = 2*cos(2*pi*500*t).*cos(2*pi*5000*t);

% Fourier Transform using FFT

X2 = fftshift(fft(x2, L));

X2mag = abs(X2)/L;

X2phase = angle(X2);

%% -------- Plot Time-domain --------

figure;

plot(t(1:1000), x2(1:1000), 'b', 'LineWidth', 1.5); % Show small window for clarity

xlabel('Time (s)'); ylabel('x_2(t)');

title('Signal x_2(t) = 2cos(2π500t)cos(2π5000t)');

grid on;

%% -------- Plot Magnitude Spectrum --------

figure;

plot(f, X2mag, 'r', 'LineWidth', 1.5);

xlabel('Frequency (Hz)'); ylabel('|X_2(f)|');

title('Magnitude Spectrum of x_2(t)');

xlim([-10000 10000]); % focus around frequencies of interest

grid on;

%% -------- Plot Phase Spectrum --------

figure;

plot(f, unwrap(X2phase), 'k', 'LineWidth', 1.5);

xlabel('Frequency (Hz)'); ylabel('Phase (rad)');

title('Phase Spectrum of x_2(t)');

xlim([-10000 10000]);

grid on;

Figure:

5.2

Sol:

clc; clear; close all;

Fs = 100e3;              % Sampling frequency (100 kHz)

T = 1/Fs;

L = 2048;                % FFT length

t = (0:L-1)*T;           % Time vector

%% (a) Rectangular pulse x(t)

x = (t >= 0 & t <= 2e-3);   % Rectangular pulse (0 to 2 ms)

% FFT

X = fftshift(fft(x, L));

f = (-L/2:L/2-1)*(Fs/L);

figure;

subplot(3,1,1);

plot(t*1000, x, 'LineWidth',1.5);

xlabel('t (ms)'); ylabel('x(t)');

title('(a) Rectangular Pulse in Time Domain'); grid on; xlim([0 5]);

subplot(3,1,2);

plot(f, abs(X)/max(abs(X)), 'LineWidth',1.5);

xlabel('Frequency (Hz)'); ylabel('|X(f)|');

title('Magnitude Spectrum of x(t)'); grid on; xlim([-5000 5000]);

subplot(3,1,3);

plot(f, angle(X), 'LineWidth',1.2);

xlabel('Frequency (Hz)'); ylabel('∠X(f)');

title('Phase Spectrum of x(t)'); grid on; xlim([-5000 5000]);

%% (b) Triangular pulse h(t)

h = zeros(size(t));

idx = (t >= 0 & t <= 1.2e-3);

h(idx) = t(idx)/(1.2e-3);   % Triangular ramp

% FFT

H = fftshift(fft(h, L));

figure;

subplot(3,1,1);

plot(t*1000, h, 'LineWidth',1.5);

xlabel('t (ms)'); ylabel('h(t)');

title('(b) Triangular Pulse in Time Domain'); grid on; xlim([0 5]);

subplot(3,1,2);

plot(f, abs(H)/max(abs(H)), 'LineWidth',1.5);

xlabel('Frequency (Hz)'); ylabel('|H(f)|');

title('Magnitude Spectrum of h(t)'); grid on; xlim([-5000 5000]);

subplot(3,1,3);

plot(f, angle(H), 'LineWidth',1.2);

xlabel('Frequency (Hz)'); ylabel('∠H(f)');

title('Phase Spectrum of h(t)'); grid on; xlim([-5000 5000]);Figure: