Signals & Communication Systems - Lab-4

Linear Convolution

Exercise 4.1: Determine analytically the convolution of the following signals, and verify

your answers using the Matlab.

x(t)=e−t⋅u(t)and h(t)=u(t)-u(t-1)

sol:

clc; clear; close all;

% time parameters

dt = 0.001;

t  = -1:dt:8;     % choose a window long enough to show response

% unit step

us = @(t,to) double(t>=to);

% signals

x = exp(-t).*us(t,0);            % x(t)=e^{-t}u(t)

h = us(t,0) - us(t,1);           % h(t)=u(t)-u(t-1) (width 1 rectangle)

% numeric convolution (scale by dt)

y_num = conv(x,h)*dt;

% time axis for convolution result: t_conv goes from t(1)+t(1) to t(end)+t(end)

t_conv = (t(1)+t(1)):dt:(t(end)+t(end));

% truncate/align to same viewing window as t

% (we'll compare on t_compare which lies inside t_conv)

t_compare =  -0.5:dt:6;  % region for comparison/plot

% analytic y(t) evaluated on t_compare

y_ana = zeros(size(t_compare));

% cases

idx1 = t_compare < 0;

idx2 = (t_compare >= 0) & (t_compare < 1);

idx3 = t_compare >= 1;

y_ana(idx1) = 0;

y_ana(idx2) = 1 - exp(-t_compare(idx2));

y_ana(idx3) = (exp(1)-1).*exp(-t_compare(idx3));  % (e-1)*e^{-t}

% extract numeric y for the same t_compare from y_num

% find indices in t_conv that correspond to t_compare

[~, idx_start] = min(abs(t_conv - t_compare(1)));

idx_end = idx_start + length(t_compare) - 1;

y_num_slice = y_num(idx_start:idx_end);

% plotting

figure('Position',[100 100 800 600]);

subplot(2,1,1);

plot(t, x,'b','LineWidth',1.5); hold on;

plot(t, h,'k','LineWidth',1.5);

legend('x(t)=e^{-t}u(t)','h(t)=u(t)-u(t-1)');

xlabel('t'); ylabel('Amplitude');

title('Signals x(t) and h(t)'); grid on;

axis([-0.5 6 -0.2 1.2]);

subplot(2,1,2);

plot(t_compare, y_num_slice,'r','LineWidth',1.5); hold on;

plot(t_compare, y_ana,'--b','LineWidth',1.5);

legend('Numerical conv (conv)','Analytical result');

xlabel('t'); ylabel('y(t)');

title('y(t)=x(t)*h(t) — numeric vs analytic'); grid on;

axis([-0.5 6 -0.05 1.2]);

% compute and display max absolute error

max_err = max(abs(y_num_slice - y_ana));

fprintf('Max absolute error between numeric and analytic y(t): %g\n', max_err);

Exercise 4.2: Consider a continuous time signal shown below. Convolve the signal itself i.e., y(t)=x(t)*x(t). Determine analytically the convolution and verify your answers using the Matlab.

Sol:

t = -1:0.001:5;

x = us(t,0)-us(t,1);

h = us(t,0)-us(t,1);h = double(h);

y = (0.002)*conv(x,h);

% y = us(t,0)-exp(-2*t).*us(t,0);

t2 = -2:0.001:10;

figure();

subplot(3,1,1);plot(t, x,'b','LineWidth',3);

xlabel('time ----->');ylabel('Amplitude ----->');

title('x(t)'); axis([-1 5 -0.2 1.2]);

subplot(3,1,2);plot(t, h,'k','LineWidth',3);

xlabel('time ----->');ylabel('Amplitude ----->');

title('h(t)');axis([-1 5 -0.2 1.2]);

subplot(3,1,3);plot(t2, y,'r','LineWidth',3);

xlabel('time ----->');ylabel('Amplitude ----->');

title('y(t)=x(t)*h(t)');axis([-1 5 -0.2 1.2]);

Figure: