Signals & Communication Systems - Lab-7

Sampling of Signals

Sol:

% Part (a): Sampling Frequency 600 Hz

clear all;

close all;

% Signal parameters

f1 = 190; % Hz

f2 = 420; % Hz

fs = 600; % Hz

Ts = 1/fs;

% Time vector for continuous signal (for plotting)

t = 0:Ts/100:0.02;

s_t = 1.2*sin(2*pi*f1*t + pi/3) + 2*cos(2*pi*f2*t - pi/5);

% Discrete time vector for sampled signal

n = 0:round(0.02/Ts);

s_n = 1.2*sin(2*pi*f1*n*Ts + pi/3) + 2*cos(2*pi*f2*n*Ts - pi/5);

% Plot the original and sampled signals

figure;

plot(t, s_t, 'b', 'DisplayName', 'Original Signal');

hold on;

stem(n*Ts, s_n, 'r', 'filled', 'DisplayName', 'Sampled Signal');

grid on;

title('Original and Sampled Signals ($f_s = 600$ Hz)');

xlabel('Time (s)');

ylabel('Amplitude');

legend;

% Frequency Spectrum

N = length(s_n);

f_fft = (-N/2:N/2-1) * (fs/N);

S_n_fft = fftshift(fft(s_n));

figure;

plot(f_fft, abs(S_n_fft), 'b');

grid on;

title('Frequency Spectrum ($f_s = 600$ Hz)');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

% The spectrum will show peaks at 190 Hz and 180 Hz (and their negative counterparts).

Figure:

B-Sol:

% Part (b): Sampling Frequency 1200 Hz

clear all;

close all;

% Signal parameters

f1 = 190; % Hz

f2 = 420; % Hz

fs = 1200; % Hz

Ts = 1/fs;

% Time vector for continuous signal (for plotting)

t = 0:Ts/100:0.02;

s_t = 1.2*sin(2*pi*f1*t + pi/3) + 2*cos(2*pi*f2*t - pi/5);

% Discrete time vector for sampled signal

n = 0:round(0.02/Ts);

s_n = 1.2*sin(2*pi*f1*n*Ts + pi/3) + 2*cos(2*pi*f2*n*Ts - pi/5);

% Plot the original and sampled signals

figure;

plot(t, s_t, 'b', 'DisplayName', 'Original Signal');

hold on;

stem(n*Ts, s_n, 'r', 'filled', 'DisplayName', 'Sampled Signal');

grid on;

title('Original and Sampled Signals ($f_s = 1200$ Hz)');

xlabel('Time (s)');

ylabel('Amplitude');

legend;

% Frequency Spectrum

N = length(s_n);

f_fft = (-N/2:N/2-1) * (fs/N);

S_n_fft = fftshift(fft(s_n));

figure;

plot(f_fft, abs(S_n_fft), 'b');

grid on;

title('Frequency Spectrum ($f_s = 1200$ Hz)');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

% The spectrum will show peaks at 190 Hz and 420 Hz (and their negative counterparts).

Figures:

Sol:

clear; close all;

f1 = 190; f2 = 420; % Hz

fs_vals = [600, 1200]; % Sampling frequencies (Hz)

Tmax = 0.02;

for k = 1:2

fs = fs_vals(k); Ts = 1/fs;

t = 0:Ts/100:Tmax;

n = 0:round(Tmax/Ts);

s_t = 1.2*sin(2*pi*f1*t + pi/3) + 2*cos(2*pi*f2*t - pi/5);

s_n = 1.2*sin(2*pi*f1*n*Ts + pi/3) + 2*cos(2*pi*f2*n*Ts - pi/5);

figure; plot(t, s_t, 'b'); hold on;

stem(n*Ts, s_n, 'r', 'filled'); grid on;

title(['Original and Sampled Signals ($f_s = ', num2str(fs), '$ Hz)'], 'Interpreter','latex');

xlabel('Time (s)'); ylabel('Amplitude');

legend('Original Signal', 'Sampled Signal');

N = length(s_n);

f_fft = (-N/2:N/2-1) * (fs/N);

S_n_fft = fftshift(fft(s_n));

figure;

plot(f_fft, abs(S_n_fft), 'b'); grid on;

title(['Frequency Spectrum ($f_s = ', num2str(fs), '$ Hz)'], 'Interpreter','latex');

xlabel('Frequency (Hz)'); ylabel('Magnitude');

end