Logical operations for AND

MOV A,#35H ;A = 35H

ANL A,#0FH ;A = A AND 0FH

; Program: Perform addition using Immediate, Register, Direct and Indirect Addressing Modes

ORG 0000H

; ---------------------------------------------------

; Immediate Addressing Mode

; ---------------------------------------------------

MOV A, #25H        ; Load first number directly into A

ADD A, #15H        ; Add second number directly

MOV 30H, A         ; Store result at RAM[30H]

; ---------------------------------------------------

; Register Addressing Mode

; ---------------------------------------------------

MOV R0, #12H       ; Load first number into R0

MOV A, R0          ; Move R0 → A

MOV R1, #14H       ; Load second number into R1

ADD A, R1          ; Add contents of R1

MOV 31H, A         ; Store result at RAM[31H]

; ---------------------------------------------------

; Direct Addressing Mode

; ---------------------------------------------------

MOV 40H, #22H      ; First number stored at RAM[40H]

MOV 41H, #11H      ; Second number stored at RAM[41H]

MOV A, 40H         ; Load first operand from RAM[40H]

ADD A, 41H         ; Add second operand from RAM[41H]

MOV 32H, A         ; Store result at RAM[32H]

; ---------------------------------------------------

; Indirect Addressing Mode

; ---------------------------------------------------

MOV R0, #50H       ; R0 points to RAM[50H]

MOV @R0, #33H      ; Store first number at RAM[50H]

INC R0             ; R0 = 51H

MOV @R0, #22H      ; Store second number at RAM[51H]

MOV R0, #50H       ; Point again to RAM[50H]

MOV A, @R0         ; Load first operand indirectly into A

INC R0             ; R0 = 51H

ADD A, @R0         ; Add second operand indirectly

MOV 33H, A         ; Store result at RAM[33H]

; ---------------------------------------------------

; End of program

; ---------------------------------------------------

SJMP $             ; Infinite loop (stop execution)

END