To Develop an Assembly language Program to perform following Arithmetic operations on 8 bit and 16-bit data Addition b) Subtraction c) Multiplication d) Division

8 bit ARITHMETIC OPERATION

org 00h

MOV AL,05H

MOV BL,02H

ADD AL,BL

MOV [5000H],AL

MOV AL,05H

MOV BL,02H

SUB AL,BL

MOV [5001H],AL

MOV AL,05H

MOV BL,02H

MUL BL

MOV [5002H],AX

MOV AL,05H

MOV BL,02H

DIV BL

MOV [5004H],AX

HLT

16 bit ARITHMETIC OPERATION

org 100h

mov ax,1234h

mov bx,5678h

add ax,bx

mov [5000h],ax

mov ax,6375h

mov bx,6005h

sub ax,bx

mov [5002h],ax

mov ax, 0502h

mov bx,0002h

mul bx

mov [5004h],ax

mov [5006h],dx

mov ax,0009h

mov dx,0000h

mov bx,0002h

div bx

mov [5008h],ax

mov [500Ah],dx

ret

Write a 8086 EMULATOR program to ADD a data byte located at offset XX00h in YYYYh segment from another data byte available at XX01h in the same segment and store the result at XX02h in the same segment?

8086 EMU8086 program that:

- Takes a data byte from memory location [YYYY:XX00H]

- Adds it to the byte at [YYYY:XX01H]

- Stores the result in [YYYY:XX02H]

🔹 8086 EMU8086 Program – Byte Addition

; Program: Add two bytes from [YYYY:XX00] and [YYYY:XX01]

;          Store result at [YYYY:XX02]

ORG 100H             ; COM file starting offset

MOV AX, YYYYH        ; Load segment address (example: 2000H or as given)

MOV DS, AX           ; DS = YYYYH

MOV SI, XX00H        ; SI = offset XX00H

MOV AL, [SI]         ; Load byte from [DS:XX00H] → AL

MOV BL, [SI+1]       ; Load byte from [DS:XX01H] → BL

ADD AL, BL           ; AL = AL + BL

MOV [SI+2], AL       ; Store result at [DS:XX02H]

HLT                  ; Stop execution

END

🔹 How to Use in EMU8086:

- Replace YYYYH with the actual segment (e.g., 2000H).

- Replace XX00H with the required offset (e.g., 0100H).

- Before running, initialize memory in Data Segment (DS = YYYYH):

  - At [YYYY:XX00H] → first number

  - At [YYYY:XX01H] → second number

- Run the program.

- The result will be at [YYYY:XX02H].

✅ Example: If YYYY = 2000H and XX00H = 0100H, then:

- 2000:0100H = 25H

- 2000:0101H = 14H
→ After execution, 2000:0102H = 39H.

Develop an 8086 Assembly language Program to perform following Logical operations on 8 bit and 16-bit data

a) AND b) OR c) NOT d) XOR

; ---------- 8-BIT OPERATIONS ----------

MOV AL, 55H       ; First 8-bit number

MOV BL, 33H       ; Second 8-bit number

0

; AND

MOV AL, 55H

AND AL, BL        ; AL = 55H AND 33H

MOV [3000H], AL   ; Store result at memory 3000H

; OR

MOV AL, 55H

OR AL, BL         ; AL = 55H OR 33H

MOV [3001H], AL

; XOR

MOV AL, 55H

XOR AL, BL        ; AL = 55H XOR 33H

MOV [3002H], AL

; NOT

MOV AL, 55H

NOT AL            ; AL = NOT 55H

MOV [3003H], AL

; ---------- 16-BIT OPERATIONS ----------

MOV AX, 1234H     ; First 16-bit number

MOV BX, 0ABCDH    ; Second 16-bit number

; AND

MOV AX, 1234H

AND AX, BX

MOV [3004H], AX   ; Store 16-bit result at 3004H

; OR

MOV AX, 1234H

OR AX, BX

MOV [3006H], AX   ; Store result at 3006H

; XOR

MOV AX, 1234H

XOR AX, BX

MOV [3008H], AX   ; Store result at 3008H

; NOT

MOV AX, 1234H

NOT AX

MOV [300AH], AX   ; Store result at 300AH

HLT               ; Stop program

END

To find the positive and negative number in an array

ORG 00H

MOV SI,5000H

MOV CL,08H 		; COUNT

MOV BL,00H 	; INITIALIZE THE RESULT BECOME 00H (COUNT FOR +VE NO)

MOV BH,00H 	; INITIALIZE THE RESULT BECOME 00H (COUNT FOR -VE NO)

LOOP:  MOV AL,[SI]

RCL AL,01H 		;( ROTATE THE ACCUMULATOR LOWER BYTE

(AL) LEFT WITH CARRY BY 1 BIT POSITION)

JC NEGATIVE  		; cy = 1 then it jumps  otherwise it goto next step

INC BL

JMP REPEAT 		; jump

NEGATIVE:INC BH 	;(BH= ADD 1 IN LSB)

REPEAT:INC SI 		; SI = 5001 AND SO ON

DEC CL 			; COUNT = 07H

JNZ LOOP  	; JUMP IF NO ZERO FOR PREVIOUS INSTRUCTION

HLT

(b) To find the EVEN and ODD number in an array

ORG 00H

MOV SI,5000H

MOV CL,08H

MOV BL,00H

MOV BH,00H

LOOP:  MOV AL,[SI]

RCR AL,01H

JC NEGATIVE

INC BL

JMP REPEAT

NEGATIVE:INC BH

REPEAT:INC SI

DEC CL

JNZ LOOP

HLT

Develop an ALP to perform Ascending/Descending order in an array

Ascending order in an array

org 100h

MOV CL,OAH

AGAIN:   	MOV DL,CL

MOV SI,4000H

BACK:    	MOV AL,[SI]

CMP AL,[SI+1]

JC NoXchg

XCHG AL,[SI+1]

XCHG AL,[SI]

NoXchg:  	INC SI

DEC DL

JNZ BACK

DEC CL

JNZ AGAIN

ret

Descending order in an array

org 100h

MOV CL,OAH

AGAIN:   	MOV DL,CL

MOV SI,4000H

BACK:    	MOV AL,[SI]

CMP AL,[SI+1]

JNC NoXchg

XCHG AL,[SI+1]

XCHG AL,[SI]

NoXchg:  	INC SI

DEC DL

JNZ BACK

DEC CL

JNZ AGAIN

ret

- Write an ALP to find largest number in a set of numbers

MOV SI,5000h

MOV AL,00H

MOV CL,0AH

Back: CMP AL, [SI]

JNC L1

MOV AL, [SI]

L1: INC SI

LOOP Back  or dec cl

MOV [5020h], AL

HLT

- Write an ALP to find smallest number in a set of numbers

MOV SI,5000h

MOV AL,00H

MOV CL,0AH

Back: CMP AL, [SI]

JC L1

MOV AL, [SI]

L1: INC SI

LOOP Back

MOV [5020h], AL

HLT